Question: The single most important step before sterilisation of an instrument is

(a) soaking it overnight

(b) drying it in a hot air oven first

(c) thorough cleaning to remove all organic matter and debris

(d) wrapping it in a fresh towel

Answer: c

Solution: ✅ Correct Answer: c. Thorough cleaning to remove all organic matter and debris.

🧠 Clinical Rationale: Residual blood, fat and tissue shield organisms from the sterilising agent, so cleaning always precedes sterilisation 🧼.

🔍 Key Clues: No cleaning, no sterilisation.

❌ Why Not Others: (d) wrapping dirty instruments protects the dirt; (a) soaking without cleaning does not remove soil; (b) heating a soiled instrument bakes the debris on.

⚡ 5-Second Recall: Clean first, always 🎯

📚 Ref: CDC disinfection guideline; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Paper-plastic pouches are used for sterilisation because they

(a) can be re-sealed after use

(b) keep items sterile indefinitely regardless of handling

(c) are impermeable to all gases

(d) allow steam or gas entry through the paper side, show an external indicator and permit aseptic opening

Answer: d

Solution: ✅ Correct Answer: d. Allow steam or gas entry through the paper side, show an external indicator and permit aseptic opening.

🧠 Clinical Rationale: The paper-plastic peel system enables steam transmission and aseptic delivery of small items 🎁.

🔍 Key Clues: Peel pack: paper in, plastic window.

❌ Why Not Others: (b) shelf life still applies; (c) impermeable packs cannot sterilise; (a) single-use pouches are never resealed.

⚡ 5-Second Recall: Peel packs for small items 🎯

📚 Ref: CSSD packaging guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The washer-disinfector is valuable because it

(a) requires no loading discipline

(b) provides reproducible cleaning and thermal disinfection with validated cycles and records

(c) replaces the autoclave entirely

(d) sterilises instruments fully

Answer: b

Solution: ✅ Correct Answer: b. Provides reproducible cleaning and thermal disinfection with validated cycles and records.

🧠 Clinical Rationale: Automated cleaning delivers consistent temperature-time disinfection and documentation, reducing manual variability 🔄.

🔍 Key Clues: Automated wash: consistent and recorded.

❌ Why Not Others: (d) cleaning and disinfection differ from sterilisation; (c) autoclaves remain essential; (a) correct loading remains critical.

⚡ 5-Second Recall: Machines reduce variability 🎯

📚 Ref: CSSD automated processing standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Flash sterilisation should be used

(a) only when no alternative exists, with immediate aseptic transfer and full documentation

(b) routinely for all instruments to save time

(c) instead of purchasing adequate instrument sets

(d) for implants as a routine final step

Answer: a

Solution: ✅ Correct Answer: a. Only when no alternative exists, with immediate aseptic transfer and full documentation.

🧠 Clinical Rationale: Flash cycles bypass packaging and sterile storage, so they are restricted to emergencies with strict traceability ⚠️.

🔍 Key Clues: Flash: emergency only.

❌ Why Not Others: (b) routine flashing increases infection risk; (c) it should not substitute for proper sets; (d) implants require validated full cycles with traceability.

⚡ 5-Second Recall: Flash only when unavoidable 🎯

📚 Ref: AORN flash sterilisation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Rigid container systems for sterilisation are advantageous because they

(a) protect instruments from damage while providing a validated barrier with filters

(b) require no cleaning before packing

(c) remove the need for instrument counts

(d) cannot be sterilised repeatedly

Answer: a

Solution: ✅ Correct Answer: a. Protect instruments from damage while providing a validated barrier with filters.

🧠 Clinical Rationale: Rigid containers combine mechanical protection with filter-mediated steam entry and organism barrier 🧰.

🔍 Key Clues: Containers protect and filter.

❌ Why Not Others: (b) cleaning remains essential; (d) containers are validated for repeated cycles; (c) counts are unrelated to packaging.

⚡ 5-Second Recall: Rigid containers protect and filter 🎯

📚 Ref: CSSD packaging systems guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Sterile packs stored correctly in a closed cupboard generally remain sterile, and shelf life is

(a) exactly seven days in all conditions

(b) infinite regardless of handling

(c) 24 hours only

(d) event-related: valid until the pack is damaged, damp or the storage conditions fail, per local policy

Answer: d

Solution: ✅ Correct Answer: d. Event-related: valid until the pack is damaged, damp or the storage conditions fail, per local policy.

🧠 Clinical Rationale: Modern practice links sterility to events such as damage or moisture rather than a fixed date, within set storage conditions 📅.

🔍 Key Clues: Event-related shelf life.

❌ Why Not Others: (a) arbitrary seven-day rules ignore storage quality; (b) damage and moisture still invalidate sterility; (c) one day is far shorter than policy allows.

⚡ 5-Second Recall: Shelf life by events, not dates 🎯

📚 Ref: CDC storage guidance; CSSD standards.

Positive Marks: 1

Negative Marks: 0.33

Question: A sterile service's "sterility assurance level" describes

(a) the expiry date of the load

(b) the probability that a validated process leaves a surviving microorganism after sterilisation

(c) the temperature of the autoclave

(d) the number of packs processed daily

Answer: b

Solution: ✅ Correct Answer: b. The probability that a validated process leaves a surviving microorganism after sterilisation.

🧠 Clinical Rationale: Sterility assurance levels express residual risk numerically (for example one in a million), linking process validation to safety 📊.

🔍 Key Clues: Assurance level: residual risk.

❌ Why Not Others: (d) throughput is activity data; (c) temperature is a cycle parameter; (a) expiry is storage dating.

⚡ 5-Second Recall: Assurance level: risk of survivors 🎯

📚 Ref: sterilisation validation standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Chlorine-based disinfectants are best suited for

(a) disinfecting fine flexible endoscopes as a sole method

(b) use on all metals for corrosion control

(c) sterilising surgical instruments before use

(d) environmental surfaces and spills at dilutions specified by policy with contact time respected

Answer: d

Solution: ✅ Correct Answer: d. Environmental surfaces and spills at dilutions specified by policy with contact time respected.

🧠 Clinical Rationale: Hypochlorites are effective, inexpensive surface and spill disinfectants but corrode metal and cannot sterilise instruments 🧴.

🔍 Key Clues: Chlorine for surfaces and spills.

❌ Why Not Others: (c) instruments need validated sterilisation; (a) endoscopes require validated high-level disinfection, often automated; (b) chlorine corrodes many metals.

⚡ 5-Second Recall: Chlorine: surfaces and spills 🎯

📚 Ref: CDC environmental disinfection.

Positive Marks: 1

Negative Marks: 0.33

Question: Manual cleaning of instruments should be performed

(a) after packing, before sterilisation

(b) under running water with vigorous splashing

(c) in a designated sink under water with PPE, brushing lumens and serrations to avoid splashing aerosols

(d) without gloves to feel the surfaces

Answer: c

Solution: ✅ Correct Answer: c. In a designated sink under water with PPE, brushing lumens and serrations to avoid splashing aerosols.

🧠 Clinical Rationale: Manual cleaning carries splash and sharps hazards, so submerged brushing with PPE protects staff and improves results 🧽.

🔍 Key Clues: Brush underwater, wear PPE.

❌ Why Not Others: (d) bare hands risk injury and contamination; (b) splashing aerosols spread organisms; (a) cleaning precedes packing.

⚡ 5-Second Recall: Clean underwater, protected 🎯

📚 Ref: CSSD manual cleaning guidance; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: Endoscopes are high-risk for cross-infection primarily because

(a) they are used on intact skin

(b) they have long narrow channels that are hard to clean and disinfect fully

(c) they are sterilised too often

(d) they are made of steel

Answer: b

Solution: ✅ Correct Answer: b. They have long narrow channels that are hard to clean and disinfect fully.

🧠 Clinical Rationale: Complex internal channels trap soil and resist disinfection, making endoscope reprocessing a documented outbreak source 🔬.

🔍 Key Clues: Narrow channels are the challenge.

❌ Why Not Others: (d) material is not the main issue; (a) they contact mucous membranes and internal surfaces; (c) repeated sterilisation is not the risk driver.

⚡ 5-Second Recall: Endoscopes: channel cleaning rules 🎯

📚 Ref: endoscope reprocessing guidelines.

Positive Marks: 1

Negative Marks: 0.33

Question: Glutaraldehyde 2 per cent achieves high-level disinfection of semi-critical items in approximately

(a) 20 minutes, with about 10 hours needed for sterilisation

(b) 30 seconds contact time

(c) 5 minutes for sterilisation

(d) 2 minutes at high-level disinfection

Answer: a

Solution: ✅ Correct Answer: a. 20 minutes, with about 10 hours needed for sterilisation.

🧠 Clinical Rationale: Glutaraldehyde is a cold chemical option: 20 minutes for high-level disinfection and a much longer soak for sterilisation 🧪.

🔍 Key Clues: 20 min HLD, 10 h sterilisation.

❌ Why Not Others: (d) and (b) are far below the required contact time; (c) sterilisation needs many hours at this concentration.

⚡ 5-Second Recall: Glutaraldehyde: 20 minutes or 10 hours 🎯

📚 Ref: CDC high-level disinfection guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The choice of sterilisation method depends primarily on

(a) the operator's preference

(b) the day of the week

(c) the item's material and design tolerance, the required turnaround and manufacturer instructions

(d) the cheapest available option only

Answer: c

Solution: ✅ Correct Answer: c. The item's material and design tolerance, the required turnaround and manufacturer instructions.

🧠 Clinical Rationale: Manufacturer instructions define validated methods for each device, and turnaround needs guide practical selection 📋.

🔍 Key Clues: Follow the manufacturer, match the material.

❌ Why Not Others: (a) preference cannot override validation; (b) scheduling does not change method adequacy; (d) cost alone compromises safety.

⚡ 5-Second Recall: Method follows the device 🎯

📚 Ref: sterilisation method selection standards; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: Items must be loaded in the autoclave so that

(a) packs are packed tightly to maximise the load

(b) steam can circulate freely with packs loosely arranged and not touching the chamber walls

(c) paper wrappers touch the chamber sides

(d) heavy items sit on top of light ones

Answer: b

Solution: ✅ Correct Answer: b. Steam can circulate freely with packs loosely arranged and not touching the chamber walls.

🧠 Clinical Rationale: Air pockets block steam, so loading follows spacing rules and never crowds the chamber toward the walls 🧺.

🔍 Key Clues: Loose loading lets steam in.

❌ Why Not Others: (a) tight packing traps air and causes failures; (d) heavy items should sit lowest; (c) touching walls prevents circulation and scorches packaging.

⚡ 5-Second Recall: Load loose, let steam flow 🎯

📚 Ref: CSSD practice standards.

Positive Marks: 1

Negative Marks: 0.33

Question: The Bowie-Dick test is used to

(a) confirm that a load is biologically sterile

(b) check chemical indicator expiry dates

(c) confirm that air removal and steam penetration are effective in a pre-vacuum autoclave

(d) test the temperature of the hot air oven

Answer: c

Solution: ✅ Correct Answer: c. Confirm that air removal and steam penetration are effective in a pre-vacuum autoclave.

🧠 Clinical Rationale: The Bowie-Dick test detects air leaks or pump faults by showing uniform colour change in a test pack, run on an empty cycle 🎯.

🔍 Key Clues: Bowie-Dick: air removal check.

❌ Why Not Others: (a) sterility is assured by biological indicators; (d) dry heat is a different method; (b) expiry checks do not test air removal.

⚡ 5-Second Recall: Bowie-Dick tests the vacuum 🎯

📚 Ref: CSSD monitoring standards; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: Packs removed from the autoclave should be

(a) cooled and dried before handling and storage, with damp packs rejected for reprocessing

(b) used immediately while hot and wet

(c) stacked tightly while still warm

(d) stored on the floor to cool

Answer: a

Solution: ✅ Correct Answer: a. Cooled and dried before handling and storage, with damp packs rejected for reprocessing.

🧠 Clinical Rationale: Hot wet packs wick organisms and can burn handlers, so cooling and drying precede storage and any damp pack is reprocessed 🌡️.

🔍 Key Clues: Cool, dry, then store.

❌ Why Not Others: (b) hot wet packs compromise sterility; (c) tight stacking traps moisture; (d) floor storage contaminates packs.

⚡ 5-Second Recall: Cool and dry before storage 🎯

📚 Ref: CSSD practice standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Ultraviolet light in theatre or CSSD is

(a) a complete steriliser for wrapped packs

(b) a substitute for autoclaves

(c) the preferred method for implants

(d) an adjunct surface and air measure, not a primary sterilising method for instruments

Answer: d

Solution: ✅ Correct Answer: d. An adjunct surface and air measure, not a primary sterilising method for instruments.

🧠 Clinical Rationale: Ultraviolet light penetrates poorly and shadows protect organisms, so it supplements cleaning rather than sterilises 📉.

🔍 Key Clues: UV: adjunct only.

❌ Why Not Others: (a) wrapped packs cannot be sterilised by UV; (c) implants need validated sterilisation; (b) autoclaves are not replaceable by UV.

⚡ 5-Second Recall: UV supplements, never sterilises packs 🎯

📚 Ref: CDC environmental guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Biological monitoring is considered the most important sterilisation monitor because it

(a) is the cheapest method

(b) can be read instantly by the operator

(c) directly tests whether the sterilising process killed highly resistant spores

(d) shows the temperature reached

Answer: c

Solution: ✅ Correct Answer: c. Directly tests whether the sterilising process killed highly resistant spores.

🧠 Clinical Rationale: Biological indicators use the toughest organisms as a proxy for sterility assurance, though results need incubation time 🧫.

🔍 Key Clues: BI: spores are the gold standard.

❌ Why Not Others: (a) cost is not the rationale; (d) temperature is shown by physical monitoring; (b) incubation takes hours, not instant.

⚡ 5-Second Recall: Biological indicators assure sterility 🎯

📚 Ref: CDC sterilisation monitoring.

Positive Marks: 1

Negative Marks: 0.33

Question: Instruments with lumens and complex joints need particular attention because

(a) air and soil hide in lumens and hinges, so cleaning, flushing and correct loading decide sterilisation success

(b) hinges are decorative

(c) they cannot be autoclaved

(d) they are usually heat-sensitive

Answer: a

Solution: ✅ Correct Answer: a. Air and soil hide in lumens and hinges, so cleaning, flushing and correct loading decide sterilisation success.

🧠 Clinical Rationale: Lumen and hinge contamination blocks steam and shields organisms, so these items get directed handling 🔧.

🔍 Key Clues: Flush the lumen, open the hinge.

❌ Why Not Others: (d) many lumened instruments are heat stable; (c) they are routinely autoclaved when clean and loaded correctly; (b) hinges are functional and contamination-prone.

⚡ 5-Second Recall: Lumens and hinges: clean inside 🎯

📚 Ref: CSSD instrument processing guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Sterilisation packaging must

(a) be reusable cardboard

(b) allow the sterilising agent to penetrate, act as a barrier to organisms afterwards and withstand the process

(c) seal items so tightly that air cannot move

(d) be completely impervious to steam

Answer: b

Solution: ✅ Correct Answer: b. Allow the sterilising agent to penetrate, act as a barrier to organisms afterwards and withstand the process.

🧠 Clinical Rationale: Packaging must let steam in while keeping organisms out and maintaining integrity after processing 📦.

🔍 Key Clues: Permeable to process, barrier to organisms.

❌ Why Not Others: (d) impervious packs never sterilise; (a) cardboard is not an approved barrier; (c) sealed air pockets prevent sterilisation.

⚡ 5-Second Recall: Packages let steam in, germs out 🎯

📚 Ref: CSSD packaging standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Ethylene oxide sterilisation is chosen for

(a) items needed within the hour

(b) cotton drapes in bulk

(c) all stainless steel instruments

(d) heat-sensitive and moisture-sensitive items that cannot withstand steam or dry heat

Answer: d

Solution: ✅ Correct Answer: d. Heat-sensitive and moisture-sensitive items that cannot withstand steam or dry heat.

🧠 Clinical Rationale: Ethylene oxide is a low-temperature gas sterilant ideal for delicate electronics, plastics and scopes, but it needs aeration 🧪.

🔍 Key Clues: Heat-sensitive: think ETO.

❌ Why Not Others: (c) steel is autoclaved efficiently; (b) linen takes steam well; (a) ETO is slow and unsuited to immediate need.

⚡ 5-Second Recall: Too fragile for steam: ETO 🎯

📚 Ref: CDC ethylene oxide guidance.

Positive Marks: 1

Negative Marks: 0.33