Question: Sharps and needles received in CSSD should be

(a) counted by touching them to verify numbers

(b) placed loosely in bins

(c) handled with puncture-resistant containers and instruments only, never by hand, with staff aware of the sharps policy

(d) sorted by hand into trays

Answer: c

Solution: ✅ Correct Answer: c. Handled with puncture-resistant containers and instruments only, never by hand, with staff aware of the sharps policy.

🧠 Clinical Rationale: Used needles and blades carry the highest injury and infection risk, so mechanical handling and the sharps policy apply ⚠️.

🔍 Key Clues: No hands on sharps.

❌ Why Not Others: (d) hand sorting causes injuries; (b) loose bins spill sharps; (a) hand counting defeats the safety purpose.

⚡ 5-Second Recall: Sharps: containers and instruments 🎯

📚 Ref: sharps safety standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Biological monitoring of steam sterilisers is typically performed

(a) never if chemical indicators look correct

(b) only when a complaint arises

(c) at least weekly, and for every load containing implants or as required by policy

(d) once a year only

Answer: c

Solution: ✅ Correct Answer: c. At least weekly, and for every load containing implants or as required by policy.

🧠 Clinical Rationale: Biological monitoring provides sterility assurance, and implant loads demand demonstrated spore kill before release 📆.

🔍 Key Clues: Weekly plus every implant load.

❌ Why Not Others: (d) annual testing misses months of risk; (b) reactive testing permits harm; (a) chemical indicators cannot substitute for biological proof.

⚡ 5-Second Recall: BI weekly and with implants 🎯

📚 Ref: CDC sterilisation monitoring; CSSD standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Emergency one-off sterilisation requests from theatre should be

(a) performed on the ward autoclave without records

(b) refused in all circumstances

(c) handled by skipping the cleaning step

(d) planned for with a designated fast pathway and logged, rather than disrupting routine loads

Answer: d

Solution: ✅ Correct Answer: d. Planned for with a designated fast pathway and logged, rather than disrupting routine loads.

🧠 Clinical Rationale: Emergency needs are anticipated with defined pathways, and cleaning steps are never skipped even when time is short ⏱️.

🔍 Key Clues: Fast path, full process.

❌ Why Not Others: (c) skipping cleaning prevents sterilisation; (b) blanket refusal endangers patients; (a) undocumented ward cycles fail traceability.

⚡ 5-Second Recall: Emergencies: fast, never shortcut 🎯

📚 Ref: CSSD workflow standards; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Failed or expired sterile packs should be

(a) removed from stock and reprocessed after inspection or discarded if unsuitable, with records updated

(b) used for minor procedures only

(c) left on the shelf with a warning sticker

(d) returned to theatre for their choice

Answer: a

Solution: ✅ Correct Answer: a. Removed from stock and reprocessed after inspection or discarded if unsuitable, with records updated.

🧠 Clinical Rationale: Non-releasable packs are never used on patients, and reprocessing or disposal keeps records accurate 🚫.

🔍 Key Clues: Out of stock, reprocess or bin.

❌ Why Not Others: (c) warning labels do not prevent use; (b) minor procedures still risk infection; (d) returning unsafe packs transfers risk.

⚡ 5-Second Recall: Expired: reprocess or discard 🎯

📚 Ref: CSSD stock management policy.

Positive Marks: 1

Negative Marks: 0.33

Question: Unannounced or unscheduled reprocessing of instruments outside CSSD should be

(a) encouraged for speed in the wards

(b) avoided, with all processing through the validated CSSD pathway except for defined, monitored exceptions

(c) allowed if staff are busy

(d) permitted with domestic pressure cookers

Answer: b

Solution: ✅ Correct Answer: b. Avoided, with all processing through the validated CSSD pathway except for defined, monitored exceptions.

🧠 Clinical Rationale: Ad hoc processing lacks validation and records, and it is a documented source of failed sterilisation ⚠️.

🔍 Key Clues: One validated pathway.

❌ Why Not Others: (a) speed does not replace validation; (d) pressure cookers are not sterilisers; (c) workload is not a justification.

⚡ 5-Second Recall: Processing belongs to CSSD 🎯

📚 Ref: CSSD policy standards; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: The decontamination area in CSSD requires

(a) no ventilation because organisms are diluted

(b) negative pressure or containment, dedicated PPE, hand hygiene facilities and a separate dirty corridor or entrance

(c) shared access with the sterile store for efficiency

(d) positive pressure to push contaminants out to the clean area

Answer: b

Solution: ✅ Correct Answer: b. Negative pressure or containment, dedicated PPE, hand hygiene facilities and a separate dirty corridor or entrance.

🧠 Clinical Rationale: Aerosol-generating decontamination work needs containment and a separate entry path to protect clean zones 🚪.

🔍 Key Clues: Dirty zone: contained and separate.

❌ Why Not Others: (d) positive pressure pushes contaminated air into the clean side; (a) ventilation is essential; (c) shared access destroys zoning.

⚡ 5-Second Recall: Dirty side contained 🎯

📚 Ref: CSSD design standards; infection control guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Closed transport containers for used instruments should be

(a) rigid, leak-proof, clearly labelled as contaminated and cleaned after each use

(b) cloth bags reused without washing

(c) cardboard boxes sealed with tape

(d) open baskets to speed drying

Answer: a

Solution: ✅ Correct Answer: a. Rigid, leak-proof, clearly labelled as contaminated and cleaned after each use.

🧠 Clinical Rationale: Rigid sealed containers protect staff and the environment from spills and aerosols during transport 🧰.

🔍 Key Clues: Sealed, labelled, cleaned.

❌ Why Not Others: (c) cardboard is not leak-proof or cleanable; (d) open baskets spill fluid and expose staff; (b) unwashed cloth bags harbour organisms.

⚡ 5-Second Recall: Rigid sealed containers 🎯

📚 Ref: CSSD transport standards.

Positive Marks: 1

Negative Marks: 0.33

Question: The CSSD technician's role in the packing zone includes

(a) cleaning the decontamination floor

(b) sterilising packs in the dirty zone

(c) issuing sterile stock to theatre without records

(d) inspecting instruments for cleanliness and function, assembling sets to the tray list and labelling packs correctly

Answer: d

Solution: ✅ Correct Answer: d. Inspecting instruments for cleanliness and function, assembling sets to the tray list and labelling packs correctly.

🧠 Clinical Rationale: Packing staff are the last quality checkpoint before sterilisation, verifying function and set composition 🔍.

🔍 Key Clues: Inspect, assemble, label.

❌ Why Not Others: (b) steriliser placement follows zoning; (c) issuing is recorded and separate; (a) floor cleaning belongs to decontamination duties.

⚡ 5-Second Recall: Packing is the last check 🎯

📚 Ref: CSSD role descriptions.

Positive Marks: 1

Negative Marks: 0.33

Question: Implant loads should be

(a) released immediately like other loads

(b) exempt from monitoring due to resource limits

(c) monitored with a biological indicator and quarantined until the result is available, unless an approved exception policy applies

(d) released without traceability records

Answer: c

Solution: ✅ Correct Answer: c. Monitored with a biological indicator and quarantined until the result is available, unless an approved exception policy applies.

🧠 Clinical Rationale: Implants carry catastrophic infection consequences, so biological proof and quarantine precede release 🦴.

🔍 Key Clues: Implants wait for BI.

❌ Why Not Others: (a) immediate release skips sterility assurance; (b) resource limits do not lower the standard; (d) implants require traceability.

⚡ 5-Second Recall: Implants: BI then release 🎯

📚 Ref: CSSD implant release standards; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: The cleaning step in CSSD most commonly combines

(a) detergent soaking with no mechanical action

(b) ultrasonic cleaning as the only step for all items

(c) only tap-water rinsing

(d) manual pre-cleaning of complex surfaces with an automated washer-disinfector cycle using enzymatic or alkaline detergent

Answer: d

Solution: ✅ Correct Answer: d. Manual pre-cleaning of complex surfaces with an automated washer-disinfector cycle using enzymatic or alkaline detergent.

🧠 Clinical Rationale: Combined manual and automated cleaning covers both gross soil and fine contamination reproducibly 🔄.

🔍 Key Clues: Manual plus machine.

❌ Why Not Others: (c) water alone removes little soil; (a) soak-only cleaning without action is inadequate; (b) some soil needs manual attention before the machine.

⚡ 5-Second Recall: Manual then machine 🎯

📚 Ref: CSSD cleaning standards.

Positive Marks: 1

Negative Marks: 0.33

Question: A CSSD audit programme should include

(a) only financial audits

(b) process observations, monitoring review, documentation checks, storage conditions and clean/sterile transport review

(c) nothing, since the department is technical

(d) staff attendance only

Answer: b

Solution: ✅ Correct Answer: b. Process observations, monitoring review, documentation checks, storage conditions and clean/sterile transport review.

🧠 Clinical Rationale: Multidimensional audits detect drift in practice and environment before failures occur 📊.

🔍 Key Clues: Observe, review, check, correct.

❌ Why Not Others: (a) financial audits are separate; (d) attendance is not a quality measure; (c) technical work still needs quality checks.

⚡ 5-Second Recall: Audit the whole CSSD chain 🎯

📚 Ref: NABH quality standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Internal chemical indicators placed inside packs mainly demonstrate

(a) the age of the instrumentation

(b) that the sterilising agent penetrated to the centre of the pack and reached required parameters

(c) the exact number of surviving spores

(d) the identity of the surgeon

Answer: b

Solution: ✅ Correct Answer: b. That the sterilising agent penetrated to the centre of the pack and reached required parameters.

🧠 Clinical Rationale: Internal indicators add penetration evidence beyond the external tape, raising confidence in the load 🧪.

🔍 Key Clues: Internal shows penetration.

❌ Why Not Others: (c) survivors are measured by biological indicators; (a) instrument age is not indicated; (d) surgeon identity is a record, not an indicator.

⚡ 5-Second Recall: Internal indicator: penetration proof 🎯

📚 Ref: CSSD monitoring standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Cleaning verification using adenosine triphosphate testing on surfaces helps to

(a) objectively assess whether manual cleaning removed organic residue before disinfection

(b) measure the temperature of the washer

(c) count the number of instruments processed

(d) sterilise the surface

Answer: a

Solution: ✅ Correct Answer: a. Objectively assess whether manual cleaning removed organic residue before disinfection.

🧠 Clinical Rationale: ATP testing gives rapid feedback on residual organic soil, validating cleaning practice 🔬.

🔍 Key Clues: ATP measures residual soil.

❌ Why Not Others: (d) ATP testing does not sterilise; (b) temperature is measured by cycle sensors; (c) instrument counts come from trays.

⚡ 5-Second Recall: ATP proves cleaning 🎯

📚 Ref: environmental cleaning verification guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: If a set arrives in theatre with an instrument missing compared with the tray list, the correct action is

(a) withhold the set from use, inform CSSD immediately and trace the missing item before the case proceeds

(b) proceed and note the shortage afterwards

(c) borrow any similar instrument from another tray

(d) continue and ask CSSD to send it next week

Answer: a

Solution: ✅ Correct Answer: a. Withhold the set from use, inform CSSD immediately and trace the missing item before the case proceeds.

🧠 Clinical Rationale: Incomplete sets mean a potential retained or lost item, so tracing takes priority over convenience 🔍.

🔍 Key Clues: Missing item: stop and trace.

❌ Why Not Others: (b) retrospective notes miss immediate risk; (c) substitution hides the discrepancy; (d) weekly follow-up loses the trail.

⚡ 5-Second Recall: Count mismatch: trace it 🎯

📚 Ref: CSSD and theatre count reconciliation standards.

Positive Marks: 1

Negative Marks: 0.33

Question: If a patient develops a suspected infection traced to an instrument set, CSSD should

(a) destroy all records to avoid blame

(b) deny any involvement without review

(c) support investigation by retrieving processing records, reviewing the cycle and assisting the infection control team

(d) continue processing without changes

Answer: c

Solution: ✅ Correct Answer: c. Support investigation by retrieving processing records, reviewing the cycle and assisting the infection control team.

🧠 Clinical Rationale: Outbreak investigation depends on transparent access to processing data, with system learning rather than defensive concealment 🔎.

🔍 Key Clues: Open the records, help the review.

❌ Why Not Others: (a) destroying records is misconduct; (b) denial blocks learning; (d) unchanged processing may propagate failures.

⚡ 5-Second Recall: Investigating? Open the records 🎯

📚 Ref: outbreak investigation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Sterile stock rotation on shelves follows

(a) random selection by whoever reaches the shelf

(b) first-expiry-first-out, with stock checked for damaged or damp packs during rotation

(c) no rotation because packs do not age

(d) last-in-first-out for convenience

Answer: b

Solution: ✅ Correct Answer: b. First-expiry-first-out, with stock checked for damaged or damp packs during rotation.

🧠 Clinical Rationale: FEFO minimises expiry wastage and rotation is the opportunity to inspect each pack 🔄.

🔍 Key Clues: Soonest expiry leaves first.

❌ Why Not Others: (d) last-in-first-out leaves short-dated stock expiring; (c) packs do age under event-related rules; (a) random picking loses control.

⚡ 5-Second Recall: FEFO every shelf 🎯

📚 Ref: CSSD stock rotation standards.

Positive Marks: 1

Negative Marks: 0.33

Question: The Bowie-Dick test is performed

(a) daily on an empty pre-vacuum steriliser chamber, before the first load, as an air-removal check

(b) after each patient

(c) weekly on a full load

(d) on every instrument set

Answer: a

Solution: ✅ Correct Answer: a. Daily on an empty pre-vacuum steriliser chamber, before the first load, as an air-removal check.

🧠 Clinical Rationale: Bowie-Dick is a daily early-morning test on an empty chamber, detecting air leaks and pump faults before clinical work 🗓️.

🔍 Key Clues: Daily, empty, first thing.

❌ Why Not Others: (c) it is not a load test; (d) it is not placed inside instrument sets; (b) it runs once daily, not per patient.

⚡ 5-Second Recall: Bowie-Dick daily, empty 🎯

📚 Ref: CSSD monitoring standards; CDC.

Positive Marks: 1

Negative Marks: 0.33

Question: Feedwater for steam sterilisers should be tested for

(a) nothing, because municipal water is always suitable

(b) colour only by eye

(c) taste before use

(d) hardness, conductivity and contaminants at defined intervals with results recorded

Answer: d

Solution: ✅ Correct Answer: d. Hardness, conductivity and contaminants at defined intervals with results recorded.

🧠 Clinical Rationale: Chemistry checks protect sterilisers and loads from scale and carryover, and the results demonstrate control 🧪.

🔍 Key Clues: Test the feedwater.

❌ Why Not Others: (b) visual checks miss dissolved contaminants; (c) tasting water is unsafe and meaningless; (a) municipal quality does not guarantee steriliser-grade water.

⚡ 5-Second Recall: Water chemistry logged 🎯

📚 Ref: steriliser water quality standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Class 1 chemical indicators such as external process tapes show

(a) the biological kill achieved

(b) that sterilisation parameters were fully met inside the pack

(c) that the pack has been exposed to the sterilisation process

(d) the storage shelf life

Answer: c

Solution: ✅ Correct Answer: c. That the pack has been exposed to the sterilisation process.

🧠 Clinical Rationale: Class 1 indicators are process indicators only, distinguishing processed from unprocessed packs 🏷️.

🔍 Key Clues: Class 1: exposure only.

❌ Why Not Others: (b) internal parameter indicators are higher classes; (a) biological kill needs BI monitoring; (d) shelf life comes from storage policy.

⚡ 5-Second Recall: Class 1 shows exposure 🎯

📚 Ref: CSSD indicator classifications.

Positive Marks: 1

Negative Marks: 0.33

Question: Validation of a new steriliser involves

(a) performance qualification only after one year of use

(b) no formal process if the vendor provides a manual

(c) a single test cycle on the first day

(d) installation, operational and performance qualification before routine use, with results documented

Answer: d

Solution: ✅ Correct Answer: d. Installation, operational and performance qualification before routine use, with results documented.

🧠 Clinical Rationale: Formal validation proves the equipment performs to specification before patients depend on it 📋.

🔍 Key Clues: IQ, OQ, PQ before use.

❌ Why Not Others: (c) one cycle does not validate; (b) manuals do not substitute for validation; (a) qualification precedes routine use, not follows.

⚡ 5-Second Recall: Validate before use 🎯

📚 Ref: sterilisation validation standards.

Positive Marks: 1

Negative Marks: 0.33