Question: A rongeur is used to

(a) bite and remove small pieces of bone during orthopaedic procedures

(b) cut suture material

(c) retract the abdominal wall

(d) clamp bleeding vessels

Answer: a

Solution: ✅ Correct Answer: a. Bite and remove small pieces of bone during orthopaedic procedures.

🧠 Clinical Rationale: Rongeurs have strong biting jaws for bone, and their jaws and springs need careful cleaning after use 🦴.

🔍 Key Clues: Rongeur bites bone.

❌ Why Not Others: (b) sutures are cut with scissors; (d) vessels are clamped with haemostats; (c) abdominal retraction uses frame retractors.

⚡ 5-Second Recall: Rongeur nibbles bone 🎯

📚 Ref: orthopaedic instrument atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: A suture sinus is

(a) an absorbable suture dissolving

(b) a keloid scar

(c) a normal healing ridge

(d) a persistent discharging tract caused by a suture acting as a foreign body, relieved by removing the offending suture

Answer: d

Solution: ✅ Correct Answer: d. A persistent discharging tract caused by a suture acting as a foreign body, relieved by removing the offending suture.

🧠 Clinical Rationale: Foreign material keeps a tract open, so removal of the suture resolves the discharge in most cases 🩹.

🔍 Key Clues: Sinus: expel the foreign body.

❌ Why Not Others: (c) healing ridges do not discharge persistently; (b) keloids are scars, not tracts; (a) dissolving sutures are absorbed, not discharged.

⚡ 5-Second Recall: Suture sinus: take it out 🎯

📚 Ref: surgical wound texts.

Positive Marks: 1

Negative Marks: 0.33

Question: The scavenging system connected to the anaesthetic machine exists to

(a) measure the end-tidal carbon dioxide concentration

(b) prevent the breathing circuit from becoming disconnected

(c) collect and remove waste anaesthetic gases so that theatre staff exposure is reduced

(d) humidify the gases delivered to the patient

Answer: c

Solution: ✅ Correct Answer: c. Collect and remove waste anaesthetic gases so that theatre staff exposure is reduced.

🧠 Clinical Rationale: Trace concentrations of volatile agents and nitrous oxide in theatre air are linked to headache, fatigue and reproductive harm, so waste gas is ducted away 🌀.

🔍 Key Clues: Scavenging protects staff, not the patient.

❌ Why Not Others: (d) humidification is a separate circuit function; (a) end-tidal carbon dioxide is measured by the capnograph; (b) disconnection is detected by the circuit alarm.

⚡ 5-Second Recall: Scavenging takes waste gas out of staff air 🎯

📚 Ref: anaesthetic gas exposure guidance; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: A basic laparotomy set typically includes

(a) anaesthesia airway equipment

(b) scalpels, scissors, artery forceps, tissue forceps, retractors, needle holders, sponge holders and towel clips

(c) only a scalpel and a needle holder

(d) dressings and suture materials only

Answer: b

Solution: ✅ Correct Answer: b. Scalpels, scissors, artery forceps, tissue forceps, retractors, needle holders, sponge holders and towel clips.

🧠 Clinical Rationale: Major abdominal surgery needs the full functional family: cutting, clamping, grasping, retracting and suturing 🏥.

🔍 Key Clues: Abdominal set: all five families.

❌ Why Not Others: (c) far too limited for laparotomy; (a) airway kit belongs to anaesthesia; (d) dressings alone cannot expose and control the abdomen.

⚡ 5-Second Recall: Laparotomy set covers five functions 🎯

📚 Ref: instrument atlases; INC curriculum.

Positive Marks: 1

Negative Marks: 0.33

Question: Plain catgut differs from chromic catgut because chromic catgut

(a) is non-absorbable

(b) is synthetic

(c) is treated with chromium salts to resist absorption longer, extending tensile strength for weeks

(d) dissolves within 24 hours

Answer: c

Solution: ✅ Correct Answer: c. Is treated with chromium salts to resist absorption longer, extending tensile strength for weeks.

🧠 Clinical Rationale: Chromic treatment slows enzymatic digestion, suiting slower-healing tissue, while both remain absorbable natural materials ⏳.

🔍 Key Clues: Chromic lasts longer.

❌ Why Not Others: (d) catgut never dissolves in a day; (b) catgut is biological, from collagen; (a) both catgut types are absorbable.

⚡ 5-Second Recall: Chromic: longer catgut 🎯

📚 Ref: suture atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: When the line isolation monitor of the operating theatre alarms during a case, the correct action is to

(a) switch off the monitor so that the alarm stops

(b) switch off all equipment in the theatre at once

(c) continue the case and deal with the alarm afterwards

(d) identify and disconnect the faulty equipment without alarming the patient or interrupting surgery unnecessarily

Answer: d

Solution: ✅ Correct Answer: d. Identify and disconnect the faulty equipment without alarming the patient or interrupting surgery unnecessarily.

🧠 Clinical Rationale: The alarm indicates increased leakage, and removing the offending device restores safety while the isolated circuit continues to protect the patient 🛠.

🚨 Priority Principle: Remove the faulty device, do not disable the safety monitor.

🔍 Key Clues: Alarm equals find and unplug the culprit.

❌ Why Not Others: (b) shutting down all equipment endangers the patient unnecessarily; (c) delaying action allows a second fault to harm the patient; (a) silencing the monitor removes the warning and the protection.

⚡ 5-Second Recall: Find the faulty device, unplug it, keep the monitor on 🎯

📚 Ref: AORN electrical safety guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The main cost driver of instrument loss in theatres is

(a) the price of detergents

(b) instruments accidentally discarded with waste or linen and unrecorded breakages or losses

(c) the number of instrument trays

(d) electricity used by autoclaves

Answer: b

Solution: ✅ Correct Answer: b. Instruments accidentally discarded with waste or linen and unrecorded breakages or losses.

🧠 Clinical Rationale: Most replacement cost comes from preventable losses, so counts and reconciliation protect the budget 💰.

🔍 Key Clues: Losses cost more than wear.

❌ Why Not Others: (a), (d) and (c) are real costs but far smaller than instrument attrition.

⚡ 5-Second Recall: Counts save money and safety 🎯

📚 Ref: theatre cost management guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Sutures used in paediatric skin closure often differ because

(a) sutures are never used in children

(b) children tolerate removal better than adults

(c) paediatric tissue requires heavy non-absorbable material

(d) rapidly absorbing, fine sutures or absorbable subcuticular closures avoid the distress of removal in children

Answer: d

Solution: ✅ Correct Answer: d. Rapidly absorbing, fine sutures or absorbable subcuticular closures avoid the distress of removal in children.

🧠 Clinical Rationale: Fast-absorbing fine sutures reduce the need for removal procedures in young patients 👶.

🔍 Key Clues: Kids favour absorbable, fine.

❌ Why Not Others: (b) removal can be distressing for children; (c) heavy material causes scarring; (a) sutures are used when needed.

⚡ 5-Second Recall: Paediatric: fine and absorbable 🎯

📚 Ref: paediatric surgery texts.

Positive Marks: 1

Negative Marks: 0.33

Question: The purpose of a Veress needle during laparoscopic surgery is to

(a) create a pneumoperitoneum by insufflating carbon dioxide into the peritoneal cavity

(b) inject local anaesthetic into the abdominal wall

(c) aspirate gastric contents before induction

(d) provide a channel for the camera cable

Answer: a

Solution: ✅ Correct Answer: a. Create a pneumoperitoneum by insufflating carbon dioxide into the peritoneal cavity.

🧠 Clinical Rationale: The spring-loaded blunt tip enters the abdomen safely and allows gas to be instilled, lifting the abdominal wall away from the viscera before the first trocar is placed 🫧.

🔍 Key Clues: Veress equals the first gas, not the first trocar.

❌ Why Not Others: (c) gastric aspiration uses a nasogastric tube; (b) local anaesthetic is given with a fine needle at the port sites; (d) the camera cable runs outside the patient through the port.

⚡ 5-Second Recall: Veress needle makes the room for the trocar 🎯

📚 Ref: minimal access surgery nursing texts; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Surgical instruments are classified by their basic function as

(a) cutting, grasping, clamping, retracting and suturing instruments

(b) metal, plastic and glass instruments

(c) large, medium and small instruments

(d) reusable and disposable only

Answer: a

Solution: ✅ Correct Answer: a. Cutting, grasping, clamping, retracting and suturing instruments.

🧠 Clinical Rationale: Function classification groups instruments by what they do, guiding selection and handling expectations 🔧.

🔍 Key Clues: Cut, grasp, clamp, retract, suture.

❌ Why Not Others: (c) and (b) describe size and material, not function; (d) describes reuse status only.

⚡ 5-Second Recall: Five functional families 🎯

📚 Ref: instrument atlases; INC OR nursing curriculum.

Positive Marks: 1

Negative Marks: 0.33

Question: A patient develops a firm, tender nodule along a healed incision months after surgery. The most likely cause is

(a) a new malignancy in all cases

(b) a normal healed scar

(c) an incisional hernia

(d) a suture granuloma or reactive nodule to retained suture material, which may need excision if symptomatic

Answer: d

Solution: ✅ Correct Answer: d. A suture granuloma or reactive nodule to retained suture material, which may need excision if symptomatic.

🧠 Clinical Rationale: Persistent suture material can provoke a granulomatous reaction, typically managed by excision when troublesome 🩹.

🔍 Key Clues: Nodule on a scar: think suture reaction.

❌ Why Not Others: (a) malignancy is uncommon and requires evaluation rather than assumption; (b) normal scars are not tender nodules; (c) hernias bulge with straining rather than forming a firm nodule.

⚡ 5-Second Recall: Suture granuloma: excise if symptomatic 🎯

📚 Ref: surgical wound texts.

Positive Marks: 1

Negative Marks: 0.33

Question: The Yankauer suction tip is most appropriately used to

(a) remove bone fragments from an open fracture

(b) decompress the stomach during laparotomy

(c) evacuate irrigant from the peritoneal cavity

(d) clear blood and secretions from the mouth and pharynx

Answer: d

Solution: ✅ Correct Answer: d. Clear blood and secretions from the mouth and pharynx.

🧠 Clinical Rationale: The rigid, bulbous, single-aperture Yankauer reaches into the oral cavity and pharynx to keep the airway clear during induction and surgery 👄.

🔍 Key Clues: Yankauer equals rigid oral suction.

❌ Why Not Others: (c) peritoneal irrigation is cleared with a Poole tip; (b) gastric decompression uses a large-bore tube; (a) bone fragments need forceps or a rongeur, not suction alone.

⚡ 5-Second Recall: Yankauer clears the airway 🎯

📚 Ref: OT nursing texts; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: The needle holder is used to

(a) cut sutures at the end of a stitch

(b) retract muscle

(c) grasp and drive the suture needle through tissue and to tie knots

(d) clamp bleeding points

Answer: c

Solution: ✅ Correct Answer: c. Grasp and drive the suture needle through tissue and to tie knots.

🧠 Clinical Rationale: Needle holders have cross-serrations and a ratchet suited to needle control and knot tying 🧵.

🔍 Key Clues: Needle holders hold needles.

❌ Why Not Others: (a) scissors cut sutures; (d) haemostats clamp vessels; (b) retractors retract.

⚡ 5-Second Recall: Needle holder drives the needle 🎯

📚 Ref: instrument atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: Adhesive skin strips (Steri-strips) are used to

(a) support well-approximated superficial wounds with low tension, often after suture or staple removal

(b) secure drains

(c) close deep fascia

(d) drain collections

Answer: a

Solution: ✅ Correct Answer: a. Support well-approximated superficial wounds with low tension, often after suture or staple removal.

🧠 Clinical Rationale: Adhesive strips provide light reinforcement for superficial closure and healing support 🩹.

🔍 Key Clues: Strips support, not replace, deep closure.

❌ Why Not Others: (c) fascia requires strong sutures; (d) drainage needs drains; (b) drains need anchored fixation.

⚡ 5-Second Recall: Strips reinforce superficial wounds 🎯

📚 Ref: wound closure guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: In bipolar electrosurgery, the current path is best described as

(a) flowing from the pencil to the patient's metal arm support

(b) flowing from the pencil to a pad on the thigh

(c) flowing from the pencil tip through the whole body back to the generator earth

(d) flowing only between the two tips of the forceps, so no return electrode is required

Answer: d

Solution: ✅ Correct Answer: d. Flowing only between the two tips of the forceps, so no return electrode is required.

🧠 Clinical Rationale: Both active and return electrodes are built into the instrument, so the circuit is short and the current does not travel through the patient's body 🩺.

🔍 Key Clues: Two tips, tiny circuit, no pad.

❌ Why Not Others: (b) a pencil and thigh pad describes monopolar diathermy; (a) contact with a metal arm support describes an accidental monopolar burn, not bipolar function; (c) whole-body conduction is the monopolar pathway.

⚡ 5-Second Recall: Bipolar current stays between the tips 🎯

📚 Ref: AORN electrosurgery guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Standardising instrument sets across the theatre complex is valuable because

(a) it removes the need for instrument counts

(b) staff recognise contents, counts are reliable, tray lists match sets and replacement and training are simpler

(c) it prevents surgeons from requesting any change forever

(d) it eliminates the need for tray lists

Answer: b

Solution: ✅ Correct Answer: b. Staff recognise contents, counts are reliable, tray lists match sets and replacement and training are simpler.

🧠 Clinical Rationale: Standard sets reduce error, speed setup and make traceability manageable 🔄.

🔍 Key Clues: Standard sets, fewer surprises.

❌ Why Not Others: (c) updates are still managed through governance; (d) tray lists remain mandatory; (a) counts are always required.

⚡ 5-Second Recall: Standard sets, standard lists 🎯

📚 Ref: theatre standardisation guidance; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: The main reason absorbable sutures with rapid absorption are chosen for mucosal closure is that

(a) mucosa heals quickly and the suture disappears as healing completes, minimising foreign material and later removal

(b) rapid sutures are cheaper

(c) rapid absorption prevents any infection

(d) mucosa cannot hold non-absorbable material

Answer: a

Solution: ✅ Correct Answer: a. Mucosa heals quickly and the suture disappears as healing completes, minimising foreign material and later removal.

🧠 Clinical Rationale: Fast mucosal healing matches rapid suture absorption, avoiding retained material in a sensitive area 👄.

🔍 Key Clues: Quick healing, quick absorb.

❌ Why Not Others: (c) no suture prevents infection by itself; (d) mucosa holds any appropriate material; (b) cost is not the clinical principle.

⚡ 5-Second Recall: Match absorption to healing 🎯

📚 Ref: suture selection principles.

Positive Marks: 1

Negative Marks: 0.33

Question: A pneumatic tourniquet must be used with the greatest caution in a patient with

(a) sickle cell disease or severe peripheral vascular disease

(b) presbyopia

(c) mild controlled hypertension

(d) a healed forearm fracture from years ago

Answer: a

Solution: ✅ Correct Answer: a. Sickle cell disease or severe peripheral vascular disease.

🧠 Clinical Rationale: Ischaemia and stasis in sickle cell disease promote sickling and vaso-occlusion, and poor peripheral perfusion already threatens the limb 🩸.

🔍 Key Clues: Sickle cells and poor circulation hate tourniquets.

❌ Why Not Others: (c) controlled hypertension is common and usually well tolerated with correct cuff pressure; (d) an old healed fracture does not affect perfusion; (b) presbyopia is a refractive eye condition with no bearing on limb ischaemia.

⚡ 5-Second Recall: Sickle cell and poor circulation mean caution 🎯

📚 Ref: Miller's Anaesthesia; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Scissors are best tested for sharpness by

(a) listening for a metallic click

(b) cutting a single layer of gauze or suture material and checking for clean cutting action

(c) visual inspection of shine only

(d) cutting thick leather

Answer: b

Solution: ✅ Correct Answer: b. Cutting a single layer of gauze or suture material and checking for clean cutting action.

🧠 Clinical Rationale: Functional testing detects blade defects that visual checks miss, and dull scissors are sent for sharpening ✂️.

🔍 Key Clues: Test the cut, not the shine.

❌ Why Not Others: (d) leather damages the blades; (c) shine does not prove sharpness; (a) click sounds do not test edges.

⚡ 5-Second Recall: Test the cut 🎯

📚 Ref: instrument inspection standards.

Positive Marks: 1

Negative Marks: 0.33

Question: A ligature differs from a suture because a ligature is

(a) always absorbable

(b) placed only during wound closure

(c) a thread tied around a vessel or duct to occlude it, whereas sutures approximate tissue

(d) used only on bone

Answer: c

Solution: ✅ Correct Answer: c. A thread tied around a vessel or duct to occlude it, whereas sutures approximate tissue.

🧠 Clinical Rationale: Ligatures occlude structures, and the same materials may serve either purpose in different forms 🩸.

🔍 Key Clues: Ligature ties off, suture brings together.

❌ Why Not Others: (a) ligatures may be absorbable or not; (d) bone uses wires and pins; (b) ligatures are also used during dissection, not only closure.

⚡ 5-Second Recall: Ligature ties, suture apposes 🎯

📚 Ref: surgical technique texts.

Positive Marks: 1

Negative Marks: 0.33

Question: Shoulder tip pain reported by a patient on the first day after laparoscopy is usually due to

(a) a pulmonary embolism

(b) residual carbon dioxide irritating the diaphragm, with pain referred along the phrenic nerve

(c) a urinary tract infection

(d) a fractured clavicle

Answer: b

Solution: ✅ Correct Answer: b. Residual carbon dioxide irritating the diaphragm, with pain referred along the phrenic nerve.

🧠 Clinical Rationale: Gas left under the diaphragm irritates it chemically, and the phrenic nerve shares the C3 to C5 spinal segments with the shoulder tip, so the pain is felt there 🫁.

🔍 Key Clues: Shoulder tip after laparoscopy equals retained gas.

❌ Why Not Others: (d) a fracture would follow trauma and is unrelated to the gas; (a) embolism causes pleuritic chest pain and breathlessness, not isolated shoulder tip pain; (c) urinary infection causes dysuria and lower abdominal pain.

⚡ 5-Second Recall: Gas under the diaphragm, pain at the shoulder tip 🎯

📚 Ref: post-anaesthesia nursing texts; ERAS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Vascular instruments such as DeBakey forceps and bulldog clamps are designed to

(a) cut bone

(b) retract the abdominal wall

(c) crush tissue for secure grip

(d) handle vessels without crushing while temporarily occluding flow during anastomosis

Answer: d

Solution: ✅ Correct Answer: d. Handle vessels without crushing while temporarily occluding flow during anastomosis.

🧠 Clinical Rationale: Atraumatic jaws and fine occlusion devices protect the endothelium during vascular repair 🩸.

🔍 Key Clues: Vascular: gentle occlusion.

❌ Why Not Others: (c) crushing would thrombose the vessel; (a) bone cutting uses osteotomes; (b) abdominal retraction uses frame retractors.

⚡ 5-Second Recall: Vascular tools protect the vessel 🎯

📚 Ref: vascular instrument guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Sutures used in ophthalmic surgery are typically

(a) heavy 2-0 braided silk

(b) absorbable 0 gauge

(c) extremely fine, such as 8-0 to 10-0, with atraumatic taper needles and minimal tissue reaction

(d) stainless steel wire

Answer: c

Solution: ✅ Correct Answer: c. Extremely fine, such as 8-0 to 10-0, with atraumatic taper needles and minimal tissue reaction.

🧠 Clinical Rationale: Ocular tissue demands microsutures with tiny needles and inert materials to protect vision 👁️.

🔍 Key Clues: Eye: fine and inert.

❌ Why Not Others: (a) and (d) and (b) are far too heavy and reactive for ocular use.

⚡ 5-Second Recall: Eye surgery: microsutures 🎯

📚 Ref: ophthalmic surgery texts.

Positive Marks: 1

Negative Marks: 0.33

Question: The safest place for the sharps container in the operating theatre is

(a) as close as practical to the point of use, at a height where it can be seen and never overfilled

(b) on the floor behind the surgeon

(c) next to the sterile instrument table, open and uncovered

(d) on the bottom shelf of the trolley, out of sight

Answer: a

Solution: ✅ Correct Answer: a. As close as practical to the point of use, at a height where it can be seen and never overfilled.

🧠 Clinical Rationale: Disposal is safest when it takes no extra steps, the opening is visible, and the container is replaced before it is three-quarters full so needles are never forced in 🗑.

🔍 Key Clues: Close, visible, never overfilled.

❌ Why Not Others: (b) a container on the floor behind staff invites injury from dropped sharps; (d) an unseen container leads to unsafe carrying of sharps; (c) an open container beside sterile instruments risks contamination and spills.

⚡ 5-Second Recall: Container at the point of use, three-quarters rule 🎯

📚 Ref: BMW Management Rules 2016; CDC sharps safety.

Positive Marks: 1

Negative Marks: 0.33

Question: Heavy instruments should not be placed loose on top of delicate ones because

(a) compression and movement bend fine tips and damage precision surfaces

(b) autoclave steam cannot reach the bottom

(c) delicate instruments need to be in the centre

(d) the tray becomes unbalanced visually

Answer: a

Solution: ✅ Correct Answer: a. Compression and movement bend fine tips and damage precision surfaces.

🧠 Clinical Rationale: Physical protection of fragile instruments prevents expensive damage and function loss 🧰.

🔍 Key Clues: Protect fine tips from weight.

❌ Why Not Others: (d) visual balance is irrelevant; (c) central placement is about access, not protection from weight; (b) steam reaches by design if loaded correctly.

⚡ 5-Second Recall: Never bury the fine tips 🎯

📚 Ref: instrument care guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Magnifying loupes are useful in fine suture work because they

(a) replace the operating microscope in all cases

(b) improve visualisation of small structures and fine suture placement, reducing tissue trauma

(c) remove the need for good lighting

(d) magnify the suture strength

Answer: b

Solution: ✅ Correct Answer: b. Improve visualisation of small structures and fine suture placement, reducing tissue trauma.

🧠 Clinical Rationale: Loupes assist precision for small vessels, nerves and fine sutures, though microscopes are used for the finest work 🔍.

🔍 Key Clues: Loupes help precision.

❌ Why Not Others: (d) optics do not change material properties; (a) microscopes remain for microsurgery; (c) lighting and optics both matter.

⚡ 5-Second Recall: Loupes sharpen the eye 🎯

📚 Ref: microsurgery technique texts.

Positive Marks: 1

Negative Marks: 0.33

Question: Which electrosurgical mode is used mainly to seal and coagulate bleeding vessels with minimal cutting of tissue?

(a) coagulation (fulguration) mode

(b) pure cut mode

(c) blend mode at maximum power

(d) bipolar cutting mode

Answer: a

Solution: ✅ Correct Answer: a. Coagulation (fulguration) mode.

🧠 Clinical Rationale: Coagulation delivers interrupted bursts of current that desiccate tissue proteins and thrombose small vessels without a continuous cutting arc 🩸.

🔍 Key Clues: Interrupted bursts seal; continuous current cuts.

❌ Why Not Others: (b) pure cut gives a continuous arc that divides tissue and causes little haemostasis; (c) high-power blend cuts and spreads thermal injury more widely; (d) bipolar cutting is not a standard generator mode for haemostasis.

⚡ 5-Second Recall: Bursts seal, continuous cuts 🎯

📚 Ref: AORN electrosurgery standards.

Positive Marks: 1

Negative Marks: 0.33

Question: All theatre staff who handle instruments should be trained to

(a) sterilise instruments on the ward

(b) repair instruments themselves

(c) prescribe instrument purchases

(d) recognise common instruments, know their proper use, handling, inspection and reporting of defects

Answer: d

Solution: ✅ Correct Answer: d. Recognise common instruments, know their proper use, handling, inspection and reporting of defects.

🧠 Clinical Rationale: Recognition and correct use prevent wrong-instrument errors and damage, and staff must report defects 🎓.

🔍 Key Clues: Know the tool, use it right.

❌ Why Not Others: (c) purchasing sits with management; (a) sterilisation belongs to CSSD; (b) repairs need engineering support.

⚡ 5-Second Recall: Know your instruments 🎯

📚 Ref: perioperative education standards.

Positive Marks: 1

Negative Marks: 0.33

Question: A suture that is completely absorbed or broken down by the body over time is classified as

(a) a ligature clip

(b) non-absorbable, such as silk and nylon

(c) a skin adhesive

(d) absorbable, such as polyglactin 910 and plain catgut

Answer: d

Solution: ✅ Correct Answer: d. Absorbable, such as polyglactin 910 and plain catgut.

🧠 Clinical Rationale: Absorbable sutures lose tensile strength and are digested or hydrolysed, so they suit buried layers that heal quickly 🧵.

🔍 Key Clues: Absorbable disappears.

❌ Why Not Others: (b) silk and nylon persist; (a) clips are metal devices, not threads; (c) adhesives are topical closure agents.

⚡ 5-Second Recall: Absorbable vanishes 🎯

📚 Ref: surgical texts; suture atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: When a laser is in use, the most important protection for everyone in the operating theatre is

(a) wavelength-specific protective eyewear for all staff, the patient's eyes taped closed and a warning sign on the door

(b) dark sunglasses worn by the surgeon only

(c) a face mask and cap for the surgical team

(d) keeping the theatre lights switched off

Answer: a

Solution: ✅ Correct Answer: a. Wavelength-specific protective eyewear for all staff, the patient's eyes taped closed and a warning sign on the door.

🧠 Clinical Rationale: Each laser wavelength damages a different ocular structure, so goggles are matched to the laser in use, the patient's eyes are taped and the door is signed to protect people entering 👓.

🚨 Priority Principle: Eye protection must match the wavelength, and the door must be marked.

🔍 Key Clues: Matched goggles, taped eyes, warning sign.

❌ Why Not Others: (b) ordinary sunglasses do not block the specific wavelength and do not protect the patient; (c) masks and caps do not protect the eyes; (d) switching off the lights does not reduce ocular exposure.

⚡ 5-Second Recall: Match the goggles to the wavelength 🎯

📚 Ref: laser safety standards; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Mixing different metals in the same instrument set is avoided because

(a) the autoclave rejects mixed loads

(b) dissimilar metals in contact with moisture cause galvanic corrosion that damages instruments

(c) the tray becomes too heavy

(d) mixed sets are hard to count

Answer: b

Solution: ✅ Correct Answer: b. Dissimilar metals in contact with moisture cause galvanic corrosion that damages instruments.

🧠 Clinical Rationale: Galvanic corrosion pits and discolours instruments, and careful segregation plus drying limits it ⚗️.

🔍 Key Clues: Unlike metals corrode each other.

❌ Why Not Others: (c) weight is not the issue; (d) counting practice is unrelated; (a) sterilisers accept loads regardless of metal mix, but damage follows.

⚡ 5-Second Recall: Keep unlike metals apart 🎯

📚 Ref: instrument care guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The key principle in choosing absorbable versus non-absorbable suture is that

(a) non-absorbable sutures are always stronger

(b) the suture must retain strength until the tissue regains its own tensile strength

(c) absorbable sutures are always safer

(d) cost should decide the choice

Answer: b

Solution: ✅ Correct Answer: b. The suture must retain strength until the tissue regains its own tensile strength.

🧠 Clinical Rationale: Matching strength-retention time to tissue healing avoids both premature wound failure and permanent foreign material ⏳.

🔍 Key Clues: Match suture life to healing time.

❌ Why Not Others: (c) and (a) are absolute statements that ignore tissue need; (d) cost never overrides clinical need.

⚡ 5-Second Recall: Strength until healed 🎯

📚 Ref: suture selection principles.

Positive Marks: 1

Negative Marks: 0.33

Question: While a tourniquet remains inflated during limb surgery, the most important nursing record is

(a) the number of sutures opened

(b) the colour of the theatre drapes

(c) the cuff pressure used and the exact time of inflation and deflation

(d) the ambient temperature of the theatre

Answer: c

Solution: ✅ Correct Answer: c. The cuff pressure used and the exact time of inflation and deflation.

🧠 Clinical Rationale: Ischaemic injury is time and pressure dependent, so the recorded inflation and deflation times let the team release the cuff before the safe limit is exceeded ⏱.

🚨 Priority Principle: Always announce and document tourniquet time aloud.

🔍 Key Clues: Pressure plus time, written down.

❌ Why Not Others: (b) drape colour has no clinical value; (a) the number of sutures is part of the count record, not the tourniquet record; (d) theatre temperature is not part of tourniquet safety.

⚡ 5-Second Recall: Tourniquet time and pressure must be written 🎯

📚 Ref: AORN tourniquet standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Artery forceps such as Mosquito and Kelly forceps are used to

(a) cut sutures

(b) clamp bleeding vessels and hold tissue securely during dissection, with the mosquito being the smallest

(c) retract the wound

(d) grasp the needle for suturing

Answer: b

Solution: ✅ Correct Answer: b. Clamp bleeding vessels and hold tissue securely during dissection, with the mosquito being the smallest.

🧠 Clinical Rationale: Haemostats differ mainly in length and jaw size, and the smallest (mosquito) suits fine vessel control 🩸.

🔍 Key Clues: Clamp to stop bleeding.

❌ Why Not Others: (a) suture cutting uses scissors; (c) retractors retract; (d) needle holders hold needles.

⚡ 5-Second Recall: Haemostats clamp vessels 🎯

📚 Ref: instrument atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: When the surgeon asks for suture cut ends to be left long, the nurse should remember that

(a) long ends prevent infection

(b) leaving ends long may be requested to allow later identification or retrieval, and the reason should be clarified and documented

(c) the request is a mistake in every case

(d) cutting ends short is always safer

Answer: b

Solution: ✅ Correct Answer: b. Leaving ends long may be requested to allow later identification or retrieval, and the reason should be clarified and documented.

🧠 Clinical Rationale: Long ends can serve specific purposes such as marking a bleeding point, and the rationale is confirmed and recorded 🔍.

🔍 Key Clues: Ask why, then document.

❌ Why Not Others: (c) the surgeon may have a valid reason; (d) short ends are usual but not always appropriate; (a) infection risk relates to material, not end length.

⚡ 5-Second Recall: Confirm the reason, then record 🎯

📚 Ref: intra-operative documentation standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Before every surgical case, each item of electrical equipment should be checked to confirm that

(a) it is the newest model available in the theatre

(b) its paintwork is free of scratches

(c) the plug, lead and casing are undamaged, the earth connection is intact and the device passes its self-test

(d) it has been used at least once in the previous week

Answer: c

Solution: ✅ Correct Answer: c. The plug, lead and casing are undamaged, the earth connection is intact and the device passes its self-test.

🧠 Clinical Rationale: Cracked leads, missing earth pins and failed self-tests are the commonest causes of shock and fire, so the check is done before the patient arrives 🔌.

🚨 Priority Principle: Equipment that fails its check is removed from service, not used.

🔍 Key Clues: Check lead, plug, earth and self-test.

❌ Why Not Others: (a) age and model do not indicate safety; (b) cosmetic scratches do not affect function; (d) recent use does not prove current safety.

⚡ 5-Second Recall: Check the lead, earth and self-test before every case 🎯

📚 Ref: NABH equipment safety standards; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: The tray list inside an instrument set is used to

(a) verify contents when assembling and again after the case, so missing items are detected early

(b) replace the intra-operative count during surgery

(c) show the price of each instrument

(d) record the patient's name before surgery begins

Answer: a

Solution: ✅ Correct Answer: a. Verify contents when assembling and again after the case, so missing items are detected early.

🧠 Clinical Rationale: Tray lists support both packing accuracy and post-case reconciliation with the surgical count 🔢.

🔍 Key Clues: Check twice against the list.

❌ Why Not Others: (c) pricing is not a clinical list; (d) patient details come from labels, not the tray list; (b) counts during surgery remain mandatory.

⚡ 5-Second Recall: Tray list on entry and exit 🎯

📚 Ref: CSSD and theatre standards.

Positive Marks: 1

Negative Marks: 0.33

Question: When selecting a suture for contaminated tissue, the preferred choice is

(a) braided silk for strength

(b) permanent braided material

(c) a monofilament material, often absorbable, and delayed primary closure may be considered for grossly contaminated wounds

(d) a skin adhesive over the contamination

Answer: c

Solution: ✅ Correct Answer: c. A monofilament material, often absorbable, and delayed primary closure may be considered for grossly contaminated wounds.

🧠 Clinical Rationale: Contaminated wounds favour low-adherence sutures, and closure timing may be delayed to allow control of infection 🦠.

🔍 Key Clues: Contaminated: monofilament, maybe delay.

❌ Why Not Others: (a) and (b) braided materials harbour bacteria; (d) sealing contamination inside causes abscess.

⚡ 5-Second Recall: Contaminated wound: smooth suture 🎯

📚 Ref: SSI prevention guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: When diathermy is required on a small appendage such as a finger or the penis, the safest technique is

(a) bipolar diathermy at low power, because a small part has a high current density with monopolar current

(b) a diathermy pad placed on the appendage itself

(c) monopolar diathermy at high power with a pad on the back

(d) monopolar diathermy with the pad wrapped around the limb

Answer: a

Solution: ✅ Correct Answer: a. Bipolar diathermy at low power, because a small part has a high current density with monopolar current.

🧠 Clinical Rationale: A narrow appendage offers a small cross-section, so monopolar current concentrates and can cause deep burns or necrosis, whereas bipolar keeps the current between the tips 🔬.

🚨 Priority Principle: Small part or confined space means choose bipolar.

🔍 Key Clues: Small tissue, high current density, use bipolar.

❌ Why Not Others: (c) high-power monopolar current in a small part can cause full-thickness necrosis; (d) wrapping the pad around a limb creates a ring burn; (b) a pad cannot be applied to the operative site itself.

⚡ 5-Second Recall: Small part, bipolar, low power 🎯

📚 Ref: AORN electrosurgery guidance; surgical texts.

Positive Marks: 1

Negative Marks: 0.33

Question: Microsurgical instruments require particular care because

(a) they can be sterilised only with ethylene oxide

(b) they do not need cleaning

(c) their fine tips are easily bent or damaged and must be protected in dedicated trays with tip guards

(d) they are cheaper to replace than repair

Answer: c

Solution: ✅ Correct Answer: c. Their fine tips are easily bent or damaged and must be protected in dedicated trays with tip guards.

🧠 Clinical Rationale: Delicate micro-instruments are stored separately with protective devices to preserve precision 🔬.

🔍 Key Clues: Micro tips: guard them.

❌ Why Not Others: (d) cost is not the reason for care; (a) many tolerate steam sterilisation; (b) all instruments need cleaning.

⚡ 5-Second Recall: Guard the fine tips 🎯

📚 Ref: microsurgical instrument care.

Positive Marks: 1

Negative Marks: 0.33

Question: Polydioxanone (PDS) is useful where prolonged absorbable support is needed because it

(a) is non-absorbable

(b) retains strength for many weeks, is a monofilament with low infection risk and is ideal for fascial closure

(c) is a braided natural material

(d) dissolves within a week

Answer: b

Solution: ✅ Correct Answer: b. Retains strength for many weeks, is a monofilament with low infection risk and is ideal for fascial closure.

🧠 Clinical Rationale: Long-lasting monofilament absorption suits slow-healing fascia, with less risk of bacterial colonisation than braided material 🧵.

🔍 Key Clues: PDS: slow, monofilament.

❌ Why Not Others: (c) PDS is synthetic monofilament; (d) rapid dissolution is not PDS behaviour; (a) PDS is absorbable.

⚡ 5-Second Recall: PDS: slow absorbing monofilament 🎯

📚 Ref: suture atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: The most important measure to prevent a patient falling during transfer from the trolley to the operating table is to

(a) ask the patient to stand and step across

(b) keep the side rails down so the team can reach the patient

(c) lock the brakes, align the two surfaces and use enough trained staff with a slide sheet

(d) move the patient quickly before anaesthesia deepens

Answer: c

Solution: ✅ Correct Answer: c. Lock the brakes, align the two surfaces and use enough trained staff with a slide sheet.

🧠 Clinical Rationale: Most transfer injuries happen when an unlocked trolley or a gap between surfaces lets the patient slide, so bracing, alignment and adequate hands are essential 🧑‍🤝‍🧑.

🚨 Priority Principle: Never transfer a sedated or anaesthetised patient with brakes off or surfaces misaligned.

🔍 Key Clues: Brakes, alignment, enough hands.

❌ Why Not Others: (a) a sedated or anaesthetised patient cannot safely step across; (d) speed does not reduce risk and increases it; (b) rails down removes protection from the very start of the move.

⚡ 5-Second Recall: Brakes on, surfaces level, team ready 🎯

📚 Ref: AORN safe patient handling.

Positive Marks: 1

Negative Marks: 0.33

Question: Babcock forceps are preferred for handling

(a) sharp needles

(b) the scalpel blade

(c) bone fragments

(d) delicate structures such as bowel, ureter and fallopian tube because the jaws are atraumatic

Answer: d

Solution: ✅ Correct Answer: d. Delicate structures such as bowel, ureter and fallopian tube because the jaws are atraumatic.

🧠 Clinical Rationale: Babcock jaws are curved and non-crushing, so tubular structures are held without injury 🩺.

🔍 Key Clues: Babcock holds tubes without crushing.

❌ Why Not Others: (a) needles are held in needle holders; (b) blades are handled with forceps; (c) bone needs bone-holding forceps.

⚡ 5-Second Recall: Babcock for tubes 🎯

📚 Ref: instrument atlases.

Positive Marks: 1

Negative Marks: 0.33

Question: Suture needles must be handled with instruments and disposed of into

(a) the linen bag

(b) the instrument tray after use

(c) the general waste bin

(d) a puncture-proof sharps container immediately after use, since they are the commonest source of needlestick injury in theatre

Answer: d

Solution: ✅ Correct Answer: d. A puncture-proof sharps container immediately after use, since they are the commonest source of needlestick injury in theatre.

🧠 Clinical Rationale: Suture needles injure more theatre staff than any other sharp, so immediate disposal and neutral zones are essential 🗑️.

🔍 Key Clues: Needle: instruments, then bin.

❌ Why Not Others: (b) needles on trays cut staff cleaning them; (a) linen bags conceal needles and injure laundry staff; (c) general waste exposes handlers.

⚡ 5-Second Recall: Needles to sharps bins, always 🎯

📚 Ref: AORN sharps safety.

Positive Marks: 1

Negative Marks: 0.33

Question: When suctioning the trachea of an intubated adult in theatre, the vacuum should be regulated so that it does not exceed approximately

(a) 80 to 120 mmHg

(b) 300 mmHg

(c) 400 mmHg

(d) 200 mmHg

Answer: a

Solution: ✅ Correct Answer: a. 80 to 120 mmHg.

🧠 Clinical Rationale: Wall vacuum is far too high for a tracheal catheter; limiting it to 80 to 120 mmHg in adults reduces mucosal trauma and hypoxia 🫁.

🔍 Key Clues: Adult airway suction equals 80 to 120 mmHg.

❌ Why Not Others: (d) 200 mmHg risks mucosal injury; (b) 300 mmHg causes bleeding and hypoxia; (c) 400 mmHg is dangerously high for tracheal suction.

⚡ 5-Second Recall: Airway suction, 80 to 120 mmHg 🎯

📚 Ref: airway management guidelines; INC OR nursing curriculum.

Positive Marks: 1

Negative Marks: 0.33

Question: Instrument lubricant or "instrument milk" is applied

(a) only to grossly damaged instruments

(b) instead of sterilisation

(c) before cleaning to loosen soil

(d) after cleaning and before sterilisation on hinges and moving parts to protect function

Answer: d

Solution: ✅ Correct Answer: d. After cleaning and before sterilisation on hinges and moving parts to protect function.

🧠 Clinical Rationale: Water-soluble lubricants keep joints smooth and reduce wear, applied after cleaning 🛢️.

🔍 Key Clues: Lubricate clean hinges.

❌ Why Not Others: (c) lubricant before cleaning traps soil; (b) it never replaces sterilisation; (a) damaged instruments are removed, not lubricated.

⚡ 5-Second Recall: Milk the hinges after cleaning 🎯

📚 Ref: instrument care standards.

Positive Marks: 1

Negative Marks: 0.33

Question: When assisting with skin closure, the nurse should have ready

(a) suction and a Yankauer tip only

(b) the planned sutures or staples, stitch scissors, toothed forceps, a sharps container and dressing materials

(c) only a dressing pack

(d) the laparotomy set unopened

Answer: b

Solution: ✅ Correct Answer: b. The planned sutures or staples, stitch scissors, toothed forceps, a sharps container and dressing materials.

🧠 Clinical Rationale: Closures run smoothly when the correct closure material, cutting instrument, sharps bin and dressing are immediately available 🧰.

🔍 Key Clues: Closure kit: suture, scissors, forceps, bin, dressing.

❌ Why Not Others: (c) a dressing alone cannot close a wound; (d) unrelated major instruments do not help skin closure; (a) suction supports the field but does not close skin.

⚡ 5-Second Recall: Closure kit ready before the last stitch 🎯

📚 Ref: AORN setup standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Which electrosurgical technique is safest for a patient with a permanent cardiac pacemaker?

(a) monopolar electrosurgery at maximum power

(b) diathermy with the return electrode placed over the pacemaker

(c) bipolar electrosurgery with short bursts and the lowest effective power

(d) monopolar electrosurgery with the pad on the chest

Answer: c

Solution: ✅ Correct Answer: c. Bipolar electrosurgery with short bursts and the lowest effective power.

🧠 Clinical Rationale: Bipolar current is confined between the two tips, so it is far less likely to interfere with pacemaker sensing or to be conducted through the myocardium 💓.

🚨 Priority Principle: Pacemaker, internal defibrillator or cochlear implant means avoid monopolar if possible.

🔍 Key Clues: Pacemaker equals prefer bipolar, keep current away from the heart.

❌ Why Not Others: (d) a chest pad routes monopolar current through the thorax and risks device inhibition; (a) maximum power increases interference and injury; (b) current across the generator can damage the device.

⚡ 5-Second Recall: Device in chest, use bipolar 🎯

📚 Ref: ASA practice advisory; AORN.

Positive Marks: 1

Negative Marks: 0.33

Question: Instruments should be returned to the storage tray after use in a countable way because

(a) count reconciliation is optional

(b) trays look neater for visitors

(c) sets must be reconciled against the tray list to detect missing items before the set leaves theatre

(d) returning them quickly speeds cleaning

Answer: c

Solution: ✅ Correct Answer: c. Sets must be reconciled against the tray list to detect missing items before the set leaves theatre.

🧠 Clinical Rationale: Immediate reconciliation closes the loop between intra-operative counts and the tray, detecting losses early ✅.

🔍 Key Clues: Reconcile before dispatch.

❌ Why Not Others: (b) appearance is not evidence; (d) cleaning flow is separate; (a) reconciliation is mandatory.

⚡ 5-Second Recall: Tray reconciled, then dispatch 🎯

📚 Ref: AORN and CSSD reconciliation standards.

Positive Marks: 1

Negative Marks: 0.33

Question: Suture size in the numbered system works so that

(a) a larger number denotes a thicker thread

(b) sizes are arbitrary

(c) a larger number denotes a thinner thread, with 4-0 thinner than 2-0

(d) size does not affect strength

Answer: c

Solution: ✅ Correct Answer: c. A larger number denotes a thinner thread, with 4-0 thinner than 2-0.

🧠 Clinical Rationale: The USP scale runs from heavy gauges to progressively finer threads, and clinicians must read sizes correctly ✅.

🔍 Key Clues: Bigger number, finer thread.

❌ Why Not Others: (a) it reverses the convention; (d) size strongly affects strength; (b) sizes are standardised, not arbitrary.

⚡ 5-Second Recall: 4-0 is finer than 2-0 🎯

📚 Ref: suture size standards.

Positive Marks: 1

Negative Marks: 0.33