Question: A characteristic advantage of pressure-controlled ventilation is

(a) no need to monitor tidal volume

(b) a guaranteed tidal volume

(c) elimination of the risk of volutrauma

(d) more even gas distribution and higher mean airway pressure for a given peak pressure, with decelerating inspiratory flow

Answer: d

Solution: ✅ Correct Answer: d. More even gas distribution and higher mean airway pressure for a given peak pressure, with decelerating inspiratory flow.

🧠 Clinical Rationale: The decelerating flow pattern improves alveolar recruitment and reduces peak pressure for the same tidal volume 🫁.

🔍 Key Clues: Pressure control: decelerating flow.

❌ Why Not Others: (b) tidal volume varies and must be monitored; (a) tidal volume monitoring is essential; (c) volutrauma risk persists if the delivered volume is excessive.

⚡ 5-Second Recall: Pressure control: decelerating flow 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The main difference between volume-controlled and pressure-controlled ventilation is that

(a) both deliver identical pressures and volumes

(b) pressure control guarantees a fixed tidal volume

(c) volume control delivers a set tidal volume with variable pressure, while pressure control delivers a set pressure with variable volume

(d) volume control delivers a set pressure with variable volume

Answer: c

Solution: ✅ Correct Answer: c. Volume control delivers a set tidal volume with variable pressure, while pressure control delivers a set pressure with variable volume.

🧠 Clinical Rationale: The controlled variable determines which parameter is guaranteed and which varies with compliance and resistance 🫁.

🔍 Key Clues: Volume control guarantees volume; pressure control guarantees pressure.

❌ Why Not Others: (d) the description is reversed; (b) pressure control gives variable volume; (a) the two modes are not equivalent.

⚡ 5-Second Recall: Volume or pressure is the guaranteed variable 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The interface choice for non-invasive ventilation is important because

(a) masks need no adjustment after fitting

(b) the mask must fit well enough to limit leak while avoiding skin breakdown and patient intolerance

(c) the largest mask is always preferred

(d) leak is desirable for comfort

Answer: b

Solution: ✅ Correct Answer: b. The mask must fit well enough to limit leak while avoiding skin breakdown and patient intolerance.

🧠 Clinical Rationale: Excessive leak reduces delivered support, while tight fitting causes pressure injury and non-adherence 🫁.

🔍 Key Clues: Fit matters for both efficacy and tolerance.

❌ Why Not Others: (c) an oversized mask leaks; (d) substantial leak reduces effectiveness; (a) ongoing adjustment and skin checks are necessary.

⚡ 5-Second Recall: Mask fit: seal without injury 🎯

📚 Ref: BTS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The plateau pressure target in lung-protective ventilation is

(a) 30 cmH2O or less

(b) 45 cmH2O or less

(c) no specific target exists

(d) 60 cmH2O or less

Answer: a

Solution: ✅ Correct Answer: a. 30 cmH2O or less.

🧠 Clinical Rationale: Limiting plateau pressure reduces alveolar over-distension and ventilator-induced lung injury 🫁.

🚨 Priority Principle: Keep plateau at or below 30.

❌ Why Not Others: (b) 45 cmH2O exceeds the safe limit; (d) 60 cmH2O is clearly injurious; (c) a specific target is well established.

⚡ 5-Second Recall: Plateau below 30 🎯

📚 Ref: ARDSNet.

Positive Marks: 1

Negative Marks: 0.33

Question: The mode that delivers a set pressure with a guaranteed minimum tidal volume by adjusting flow is

(a) continuous mandatory ventilation

(b) airway pressure release ventilation

(c) pressure support ventilation

(d) pressure-regulated volume control

Answer: d

Solution: ✅ Correct Answer: d. Pressure-regulated volume control.

🧠 Clinical Rationale: Pressure-regulated volume control combines the advantages of pressure control with a target tidal volume 🫁.

🔍 Key Clues: A hybrid mode with a volume target.

❌ Why Not Others: (a) continuous mandatory ventilation delivers a set volume with no such guarantee mechanism; (b) airway pressure release ventilation cycles between two pressure levels; (c) pressure support gives no volume guarantee.

⚡ 5-Second Recall: Pressure-regulated volume control: pressure with a volume target 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: During a ventilator check, the nurse should confirm

(a) that alarms are silenced

(b) settings match the prescription, alarms are active, tubing is intact and the humidifier is functioning

(c) that the patient is asleep

(d) only that the ventilator is switched on

Answer: b

Solution: ✅ Correct Answer: b. Settings match the prescription, alarms are active, tubing is intact and the humidifier is functioning.

🧠 Clinical Rationale: Systematic checks detect discrepancies and equipment failures before harm occurs 🫁.

🚨 Priority Principle: Verify settings against the prescription.

❌ Why Not Others: (d) power alone is insufficient; (a) silenced alarms remove a key safety net; (c) sleep is not a verification parameter.

⚡ 5-Second Recall: Ventilator check: settings, alarms, tubing, humidity 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: A spontaneous breathing trial is typically conducted with

(a) minimal support and low positive end-expiratory pressure for 30 to 120 minutes with close monitoring

(b) full ventilatory support for 24 hours

(c) no monitoring

(d) the patient fully paralysed

Answer: a

Solution: ✅ Correct Answer: a. Minimal support and low positive end-expiratory pressure for 30 to 120 minutes with close monitoring.

🧠 Clinical Rationale: A brief trial of minimal support predicts successful extubation while allowing rapid return to support if it fails 🫁.

🔍 Key Clues: Spontaneous breathing trial: 30 to 120 minutes.

❌ Why Not Others: (b) full support does not test spontaneous capacity; (d) paralysis prevents spontaneous breathing; (c) monitoring is essential throughout.

⚡ 5-Second Recall: Spontaneous breathing trial: short and monitored 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: A patient on non-invasive ventilation has a large persistent leak around the mask. The nurse should

(a) reposition or change the interface and reassess synchrony, tidal volume and comfort

(b) increase the inspiratory pressure indefinitely

(c) strap the mask more tightly until the leak stops at all costs

(d) abandon non-invasive ventilation immediately

Answer: a

Solution: ✅ Correct Answer: a. Reposition or change the interface and reassess synchrony, tidal volume and comfort.

🧠 Clinical Rationale: Correcting the interface restores effective support without causing skin injury 🫁.

🔍 Key Clues: Fix the fit before increasing pressure.

❌ Why Not Others: (b) raising pressure alone does not solve leaks; (c) excessive strap tension causes pressure ulcers; (d) abandonment is premature before correcting the interface.

⚡ 5-Second Recall: Leak: fix the interface 🎯

📚 Ref: BTS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Which statement about humidification during mechanical ventilation is correct

(a) humidification is unnecessary for intubated patients

(b) humidification increases the risk of infection without benefit

(c) heated humidification or a heat and moisture exchanger preserves mucosal function and prevents secretion retention and tube occlusion

(d) cold dry gas is preferred

Answer: c

Solution: ✅ Correct Answer: c. Heated humidification or a heat and moisture exchanger preserves mucosal function and prevents secretion retention and tube occlusion.

🧠 Clinical Rationale: Bypassing the upper airway requires artificial humidification to keep secretions manageable 🫁.

🔍 Key Clues: Intubated patients need humidity.

❌ Why Not Others: (a) humidification is essential; (d) dry cold gas thickens secretions; (b) adequate humidification reduces, rather than increases, plugging risk.

⚡ 5-Second Recall: Ventilated: humidify the gas 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Recruitment manoeuvres in ARDS

(a) are contraindicated in all patients

(b) have no risk

(c) transiently raise airway pressure to open collapsed alveoli and may improve oxygenation, but require monitoring for hypotension

(d) replace lung-protective ventilation

Answer: c

Solution: ✅ Correct Answer: c. Transiently raise airway pressure to open collapsed alveoli and may improve oxygenation, but require monitoring for hypotension.

🧠 Clinical Rationale: Recruitment raises intrathoracic pressure and can reduce venous return, so haemodynamics are monitored 🫁.

🚨 Priority Principle: Recruitment: watch the blood pressure.

❌ Why Not Others: (a) they have a role in selected patients; (d) lung-protective ventilation remains the foundation; (b) hypotension and barotrauma are recognised risks.

⚡ 5-Second Recall: Recruitment: open alveoli, watch pressure 🎯

📚 Ref: ESICM ARDS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Continuous positive airway pressure in a spontaneously breathing patient

(a) guarantees a tidal volume

(b) maintains positive airway pressure throughout the respiratory cycle, improving oxygenation and reducing work of breathing

(c) provides full ventilatory support with mandatory breaths

(d) is used only during cardiac arrest

Answer: b

Solution: ✅ Correct Answer: b. Maintains positive airway pressure throughout the respiratory cycle, improving oxygenation and reducing work of breathing 🫁.

🧠 Clinical Rationale: Positive pressure throughout the cycle recruits alveoli and offsets intrinsic positive end-expiratory pressure 🫁.

🔍 Key Clues: CPAP: pressure without mandatory breaths.

❌ Why Not Others: (c) CPAP provides no mandatory breaths; (a) tidal volume depends on the patient; (d) it is not used during cardiac arrest.

⚡ 5-Second Recall: CPAP: pressure, no mandatory breaths 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The first nursing action on a high-pressure ventilator alarm in a deteriorating patient is

(a) assess the patient, then check for tube obstruction, kinking, biting, secretions and pneumothorax

(b) reduce the positive end-expiratory pressure

(c) silence the alarm and document

(d) increase the pressure limit

Answer: a

Solution: ✅ Correct Answer: a. Assess the patient, then check for tube obstruction, kinking, biting, secretions and pneumothorax.

🧠 Clinical Rationale: A high-pressure alarm indicates resistance or compliance problems that may be immediately life-threatening 🫁.

🚨 Priority Principle: Alarm: look at the patient first.

❌ Why Not Others: (c) silencing and documenting without assessment risks harm; (d) raising the limit does not treat the cause; (b) reducing positive end-expiratory pressure does not address obstruction or a pneumothorax.

⚡ 5-Second Recall: High pressure: patient first 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The nurse should suspect non-invasive ventilation failure when

(a) the carbon dioxide falls

(b) the respiratory rate, acidosis or consciousness fail to improve within the first 1 to 2 hours

(c) the oxygen saturation improves

(d) the patient reports comfort

Answer: b

Solution: ✅ Correct Answer: b. The respiratory rate, acidosis or consciousness fail to improve within the first 1 to 2 hours.

🧠 Clinical Rationale: Early assessment identifies non-responders so that intubation is not dangerously delayed 🫁.

🚨 Priority Principle: Reassess at 1 to 2 hours; failure means escalate.

❌ Why Not Others: (d) comfort suggests tolerance; (a) falling carbon dioxide indicates success; (c) improved saturation indicates benefit.

⚡ 5-Second Recall: Non-invasive ventilation: reassess at 1 to 2 hours 🎯

📚 Ref: BTS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The primary indication for initiating invasive mechanical ventilation in the ICU is

(a) a respiratory rate above 20 per minute alone

(b) any patient admitted with pneumonia

(c) failure of oxygenation or ventilation, or a need to protect the airway, that cannot be managed non-invasively

(d) an oxygen saturation of 93 per cent

Answer: c

Solution: ✅ Correct Answer: c. Failure of oxygenation or ventilation, or a need to protect the airway, that cannot be managed non-invasively.

🧠 Clinical Rationale: Ventilation supports gas exchange and airway protection when spontaneous breathing is inadequate 🫁.

🔍 Key Clues: Ventilate when breathing or the airway fails.

❌ Why Not Others: (a) a rate of 20 alone does not require ventilation; (d) mild desaturation is managed with oxygen; (b) not every pneumonia needs invasive ventilation.

⚡ 5-Second Recall: Ventilate for failure of breathing or airway 🎯

📚 Ref: ICU ventilation guidance; ARDSNet.

Positive Marks: 1

Negative Marks: 0.33

Question: Which of the following best describes the goal of ventilation in a patient with severe metabolic acidosis and respiratory compensation

(a) reducing the respiratory rate to normal

(b) aiming for a normal carbon dioxide level regardless of pH

(c) supporting the patient's high minute ventilation while treating the underlying acidosis

(d) sedating the patient to reduce breathing

Answer: c

Solution: ✅ Correct Answer: c. Supporting the patient's high minute ventilation while treating the underlying acidosis.

🧠 Clinical Rationale: The compensatory hyperventilation is physiological, and suppressing it worsens acidaemia 🫁.

🚨 Priority Principle: Metabolic acidosis: do not blunt compensation.

❌ Why Not Others: (a) slowing the rate worsens acidosis; (b) normalising carbon dioxide ignores compensation; (d) sedation blunts the compensatory response.

⚡ 5-Second Recall: Acidosis: support the compensation 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: Which combination correctly represents a lung-protective strategy

(a) tidal volume 12 mL per kg, plateau 40 cmH2O, low positive end-expiratory pressure

(b) tidal volume 10 mL per kg, no plateau limit

(c) tidal volume 6 mL per kg with no monitoring of pressures

(d) tidal volume 6 mL per kg predicted body weight, plateau pressure 30 cmH2O or less, and appropriate positive end-expiratory pressure

Answer: d

Solution: ✅ Correct Answer: d. Tidal volume 6 mL per kg predicted body weight, plateau pressure 30 cmH2O or less, and appropriate positive end-expiratory pressure.

🧠 Clinical Rationale: Combining low tidal volume, pressure limitation and adequate positive end-expiratory pressure reduces ventilator-induced lung injury 🫁.

🚨 Priority Principle: Volume, pressure and recruitment together.

❌ Why Not Others: (a) high volume and high pressure are injurious; (b) no plateau limit risks over-distension; (c) monitoring pressures is essential for safety.

⚡ 5-Second Recall: Lung protection: 6, 30 and adequate positive end-expiratory pressure 🎯

📚 Ref: ARDSNet.

Positive Marks: 1

Negative Marks: 0.33

Question: A heat and moisture exchanger should be used with caution in a patient with

(a) any patient on volume control

(b) a tracheostomy only

(c) a normal respiratory pattern

(d) copious or bloody secretions, or a low tidal volume, because it adds dead space and may occlude

Answer: d

Solution: ✅ Correct Answer: d. Copious or bloody secretions, or a low tidal volume, because it adds dead space and may occlude.

🧠 Clinical Rationale: A heat and moisture exchanger adds resistance and dead space, so heavy secretions or small tidal volumes warrant heated humidification 🫁.

🔍 Key Clues: Heavy secretions: avoid a heat and moisture exchanger.

❌ Why Not Others: (a) volume control alone is not a contraindication; (c) a normal pattern poses no problem; (b) a tracheostomy does not itself contraindicate its use.

⚡ 5-Second Recall: Wet or bloody secretions: use heated humidification 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The inspiratory to expiratory ratio of 1 to 2 in a ventilated patient

(a) allows adequate expiratory time and reduces the risk of gas trapping

(b) applies only in pressure support

(c) is used to increase mean airway pressure

(d) is the same as inverse ratio ventilation

Answer: a

Solution: ✅ Correct Answer: a. Allows adequate expiratory time and reduces the risk of gas trapping.

🧠 Clinical Rationale: A longer expiratory phase supports complete exhalation in most patients 🫁.

🔍 Key Clues: 1 to 2: expiration longer than inspiration.

❌ Why Not Others: (c) a shorter expiratory time would raise mean airway pressure; (d) inverse ratio reverses the proportion; (b) the ratio applies to controlled modes too.

⚡ 5-Second Recall: Ratio 1 to 2: expiratory time longer 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The recommended duration of prone positioning in severe ARDS is

(a) 4 hours per session

(b) 1 hour per session

(c) continuous for 48 hours

(d) at least 12 to 16 hours per session

Answer: d

Solution: ✅ Correct Answer: d. At least 12 to 16 hours per session.

🧠 Clinical Rationale: Sustained proning is required to gain the oxygenation and survival benefit, with careful pressure-area care 🫁.

🔍 Key Clues: Proning: long sessions, 12 to 16 hours.

❌ Why Not Others: (b) an hour is too short to gain benefit; (a) 4 hours is insufficient; (c) uninterrupted proning for 48 hours without breaks risks complications.

⚡ 5-Second Recall: Prone 12 to 16 hours 🎯

📚 Ref: ESICM ARDS guidance.

Positive Marks: 1

Negative Marks: 0.33

Question: The nurse notices the ventilator delivering a much larger tidal volume than set in volume-controlled mode. The most likely explanation is

(a) the oxygen concentration is too high

(b) a leak or the ventilator's inspiratory flow pattern, or a measurement artefact requiring patient assessment and equipment check

(c) the patient is not connected

(d) the alarms are off

Answer: b

Solution: ✅ Correct Answer: b. A leak or the ventilator's inspiratory flow pattern, or a measurement artefact requiring patient assessment and equipment check.

🧠 Clinical Rationale: Discrepancies between set and measured values indicate a circuit, patient or sensor problem that must be resolved 🫁.

🚨 Priority Principle: Set versus measured mismatch: check the circuit.

❌ Why Not Others: (c) a disconnected circuit would show low, not high, values; (a) oxygen concentration does not change volume; (d) alarm status does not explain a volume discrepancy.

⚡ 5-Second Recall: Mismatch: check circuit and sensor 🎯

📚 Ref: ICU ventilation guidance.

Positive Marks: 1

Negative Marks: 0.33