Question: A used surgical instrument has visible blood but no further processing yet. A trainee says disinfection and cleaning are the same step. What is the correct distinction?

(a) Cleaning is done after sterilization to remove chemical residues only

(b) Disinfection removes visible soil; cleaning kills spores

(c) Both terms mean sterilization and can be used interchangeably

(d) Cleaning is physical removal of soil; disinfection kills most vegetative microbes but not necessarily spores, and cleaning must always precede disinfection/sterilization

Answer: d

Solution: ✅ Cleaning = removal of organic/inorganic soil; disinfection = killing most vegetative pathogens (not reliably spores); clean first or disinfection/sterilization fails.

🧠 Blood and protein shield microbes and inactivate germicides — without cleaning, even autoclaving can fail.

❌ Reversing definitions or treating disinfection as sporicidal overstates its power; post-sterilization cleaning is not the process.

⚡ NORCET Fact: Universal order — Clean → Disinfect/Sterilize; antisepsis = on living tissue, disinfection = on inanimate objects.

Positive Marks: 1

Negative Marks: 0

Question: Spirit is applied to intact skin before venepuncture, while the trolley top is wiped with sodium hypochlorite. Which term pair is correct?

(a) Both are sterilization

(b) Skin preparation is antisepsis; trolley decontamination is disinfection

(c) Both are antisepsis

(d) Skin preparation is disinfection; trolley wiping is antisepsis

Answer: b

Solution: ✅ Antiseptics are for living tissue (skin/mucosa); disinfectants are for inanimate surfaces/objects.

🧠 Same agent can be antiseptic or disinfectant depending on site and concentration — site defines the term.

❌ Calling skin prep disinfection or surface wiping antisepsis reverses Spaulding-adjacent definitions examiners love.

⚡ NORCET Fact: Antisepsis examples — chlorhexidine, povidone-iodine, 70% alcohol on skin; disinfection — hypochlorite, phenolics on surfaces.

Positive Marks: 1

Negative Marks: 0

Question: A cardiac implant, a flexible endoscope and a blood-pressure cuff are to be classified by Spaulding. Which classification is correct?

(a) All three are critical items

(b) Implant-semicritical, endoscope-noncritical, BP cuff-critical

(c) Implant-noncritical, endoscope-critical, BP cuff-semicritical

(d) Implant-critical, endoscope-semicritical, BP cuff-noncritical

Answer: d

Solution: ✅ Critical enters sterile tissue/vasculature (implant → sterilize); semicritical contacts mucosa/non-intact skin (endoscope → HLD minimum); noncritical touches intact skin only (cuff → low-level disinfection).

🧠 Risk = invasiveness of contact site; higher tissue penetration demands higher lethality level.

❌ Upgrading the cuff or downgrading the implant to same level violates risk-based reprocessing.

⚡ NORCET Fact: Spaulding trio — Critical-Sterilize, Semicritical-HLD, Noncritical-LLD; laryngoscope blade = semicritical.

Positive Marks: 1

Negative Marks: 0

Question: A laryngoscope blade and a vaginal speculum used in an OPD need reprocessing. What minimum level is required?

(a) High-level disinfection after thorough cleaning, or sterilization where feasible

(b) Sterilization is never acceptable for semicritical items

(c) Simple rinsing with tap water is enough

(d) Low-level disinfection only

Answer: a

Solution: ✅ Both contact mucous membranes → semicritical → minimum high-level disinfection after cleaning; sterilization is ideal if material-compatible.

🧠 Mucosa lacks intact keratin barrier — vegetative bacteria, mycobacteria and most viruses must all be killed.

❌ Tap-water rinse or LLD leaves mycobacteria/viruses viable; sterilization remains acceptable, not prohibited.

⚡ NORCET Fact: Endoscopes, bronchoscopes, diaphragm fitting rings, anaesthesia breathing circuits = semicritical → HLD.

Positive Marks: 1

Negative Marks: 0

Question: Which statement correctly differentiates high-, intermediate- and low-level disinfection (HLD/ILD/LLD)?

(a) ILD kills spores but HLD does not

(b) HLD kills all vegetative organisms + mycobacteria + most fungi/viruses but not necessarily bacterial spores; ILD kills vegetative + most fungi + TB bacilli but not spores; LLD kills most vegetative bacteria + some fungi/viruses, not TB or spores

(c) All three levels kill bacterial spores equally

(d) LLD is stronger than HLD and can replace sterilization

Answer: b

Solution: ✅ HLD spares only heavy spore loads; ILD reliably kills M. tuberculosis among vegetative spectrum; LLD is narrowest, missing TB and spores.

🧠 Spectrum hierarchy — HLD > ILD > LLD; spore-killing separates sterilization from even HLD.

❌ Equating levels or inverting HLD/LLD strength is a classic distractor.

⚡ NORCET Fact: 2% glutaraldehyde (≥20 min), OPA, H2O2 = HLD; hypochlorite/phenolics = ILD; quats = LLD for noncritical surfaces.

Positive Marks: 1

Negative Marks: 0

Question: A flexible duodenoscope that cannot tolerate heat is available for ERCP. Which reprocessing choice is correct?

(a) Soak briefly in saline without cleaning before HLD

(b) Low-level disinfection with quaternary ammonium spray only

(c) Meticulous cleaning + leak test + high-level disinfection with 2% glutaraldehyde/OPA per IFU with adequate contact time and rinsing

(d) Wipe with dry cloth and reuse directly

Answer: c

Solution: ✅ Heat-labile semicritical scopes need cleaning + leak testing + HLD with approved agent, concentration, time, temperature and post-HLD sterile-water rinse.

🧠 Elevator channels trap bioburden — skipping cleaning or shortening contact time causes CRE outbreaks.

❌ Dry wipe, quat spray or HLD without cleaning cannot achieve high-level kill in lumens.

⚡ NORCET Fact: Duodenoscope risk — audit HLD concentration with test strips every cycle/day; track scope-to-patient traceability.

Positive Marks: 1

Negative Marks: 0

Question: Wrapped surgical linen packs are loaded into a steam sterilizer at 121°C, 15 psi for 15 minutes. Which principle explains sterilization here?

(a) Ionizing radiation breaking DNA without heat

(b) Dry oxidation of spores by hot air

(c) Filtration physically removing microbes from the linen

(d) Moist heat in the form of saturated steam under pressure coagulates and denatures microbial proteins, including spores

Answer: d

Solution: ✅ Saturated steam under pressure transfers latent heat rapidly, denaturing proteins and killing all microbes including Geobacillus spores.

🧠 Pressure only raises the boiling point to achieve sporicidal temperatures; moisture is the lethal agent, not pressure per se.

❌ Dry heat, radiation and filtration work by different mechanisms and equipment.

⚡ NORCET Fact: Standard steam cycle — 121°C/15 psi/15 min or 134°C/30 psi/3–10 min (porous vs flash cycles vary by load).

⚡ NORCET Fact: Standard steam cycle — 121°C/15 psi/15 min or 134°C/30 psi/3–10 min (porous vs flash cycles vary by load).

Positive Marks: 1

Negative Marks: 0

Question: After a steam cycle the nurse finds packs are wet and the pressure gauge showed air pockets. Which loading error most likely caused sterilization failure?

(a) Using distilled water instead of tap water

(b) Placing packs loosely with adequate spacing

(c) Overloading with no steam circulation space, mixing textile with metal bowls incorrectly, and trapping air without proper air removal

(d) Keeping the door gasket clean

Answer: c

Solution: ✅ Overpacking blocks steam penetration; trapped air insulates load centres; mixed loads need correct placement (textiles above, metal below) with air evacuation.

🧠 Steam must contact every surface — air is the enemy of steam sterilization; wet packs wick contaminants post-cycle.

❌ Distilled water is preferred; clean gaskets and loose spacing aid, not hinder, sterilization.

⚡ NORCET Fact: Reject wet, torn or dropped packs — consider unsterile; allow drying/cooling before handling to avoid wicking.

Positive Marks: 1

Negative Marks: 0

Question: During a power cut the CSSD runs a flash (immediate-use) steam cycle for an urgently needed orthopaedic screw set. Which statement is correct?

(a) No monitoring or documentation is needed for flash cycles

(b) Flash-sterilized items can be stored for later use like routine packs

(c) Immediate-use steam sterilization is only for unwrapped, urgently needed, non-implant items that will be used immediately, with documentation; implants need biological monitoring and avoid flash if possible

(d) Flash sterilization is routine for implants and wrapped storage

Answer: c

Solution: ✅ Immediate-use = unwrapped, no storage, immediate use, fully monitored and documented; avoid for implants unless no alternative with BI + quarantine.

🧠 No protective wrap means rapid recontamination — storage defeats the purpose and hides risk.

❌ Routine flash use or storing flashed items normalises an exception meant for emergencies.

⚡ NORCET Fact: NORCET trap — flash sterilization is NOT for routine, implantable or ophthalmic loads.

Positive Marks: 1

Negative Marks: 0

Question: Glass syringes, anhydrous oils and sharp cutting instruments that corrode in steam need sterilization. Which method and parameter is correct?

(a) Boiling for 10 minutes achieves sterilization

(b) UV light exposure for 5 minutes sterilizes oils

(c) Hot-air oven (dry heat) at 160°C for 2 hours or 170°C for 1 hour

(d) Steam at 121°C for 15 minutes is still the best choice

Answer: c

Solution: ✅ Dry heat oxidises cells/spores; standard — 160°C/2 hr or 170°C/1 hr (plus heating/cooling time) suits water-impermeable oils, powders, glass and sharps.

🧠 No moisture means higher temperature + longer time needed versus steam; ideal where steam cannot penetrate or corrodes.

❌ Boiling and UV are disinfection-level only and cannot sterilize oils or lumens.

⚡ NORCET Fact: Dry-heat BI — Bacillus atrophaeus (subtilis) spores; never use dry heat for fabrics, plastics or aqueous solutions.

Positive Marks: 1

Negative Marks: 0

Question: A heat-sensitive endoscope and a metal tray contaminated with hepatitis B need chemical processing. Which agent choice is correct?

(a) 2% glutaraldehyde/OPA provides high-level disinfection/chemosterilization with prolonged contact; peracetic acid and hydrogen-peroxide plasma sterilize heat-labile items

(b) Formalin tablets at room temperature instantly sterilize without aeration

(c) Phenol is the sterilant of choice for endoscopes

(d) Savlon (chlorhexidine + cetrimide) alone sterilizes all instruments regardless of contact time

Answer: a

Solution: ✅ Aldehyde/oxidiser chemistry (glutaraldehyde, OPA, peracetic acid, H2O2 gas plasma, ethylene oxide) achieves HLD/sterilization for heat-labile items per concentration-time rules.

🧠 Sporicidal action needs full concentration + contact time + pH/temperature control — shortcuts leave HBV/HCV viable.

❌ Savlon and phenols are antiseptic/disinfectant level, not sterilants; ETO needs long aeration for toxicity.

⚡ NORCET Fact: ETO sterilizes plastics/catheters but needs 8–12 hr aeration; H2O2 plasma is fast with no toxic residue but lumen-length limits.

Positive Marks: 1

Negative Marks: 0

Question: After manual cleaning, a bronchoscope channel is soaked in 2% glutaraldehyde for only 5 minutes instead of 20–45 minutes. What is the consequence?

(a) Upgrades the process to sterilization automatically

(b) Achieves sterilization faster with no downside

(c) Contact time does not matter if solution is fresh

(d) Fails to achieve HLD — surviving mycobacteria, non-enveloped viruses and heavy bioburden risk next-patient infection; requires repeat full HLD cycle

Answer: d

Solution: ✅ HLD needs validated concentration + minimum effective contact time (commonly ≥20 min for glutaraldehyde, ≥12 min for OPA per IFU); truncation = disinfection failure.

🧠 Mycobacteria and non-enveloped viruses need sustained alkylation/oxidation — 5 minutes covers only low-level vegetative kill.

❌ Fresh solution cannot compensate for inadequate time; longer chemosterilization (≥10 hr glutaraldehyde) is a different claim.

⚡ NORCET Fact: Verify with MEC test strips, timer, temperature log and thorough pre-cleaning every single cycle.

Positive Marks: 1

Negative Marks: 0

Question: Pre-packed plastic IV fluid bottles and a burns-unit air supply need microbial control without heat damage. Which combination is correct?

(a) Autoclave the plastic bottles at 160°C dry heat and boil the air

(b) Gamma irradiation for pre-packed disposables; HEPA filtration for air supply — both are non-thermal physical methods

(c) UV irradiation sterilizes the sealed IV fluids completely

(d) Membrane filtration sterilizes the room air by autoclaving

Answer: b

Solution: ✅ Ionizing (gamma/e-beam) radiation sterilizes sealed disposables (syringes, catheters, IV sets); HEPA (0.3 μm) filtration sterilizes air and heat-labile liquids by physical removal.

🧠 Both avoid heat/moisture damage — radiation breaks DNA, filters trap microbes without adding chemicals.

❌ Dry heat destroys plastics/fluids; UV penetrates poorly and cannot sterilize sealed fluids; filters do not autoclave air.

⚡ NORCET Fact: Non-ionizing UV = surface disinfection only; ionizing gamma = true sterilization of disposables; 0.22 μm membrane = cold sterilization of fluids.

Positive Marks: 1

Negative Marks: 0

Question: A CSSD technician places a Bowie-Dick test pack, a Helix chemical indicator and a Geobacillus stearothermophilus vial in respective cycles. What does each monitor?

(a) Bowie-Dick checks dry-heat efficiency; Geobacillus checks radiation dose

(b) All three only check the colour of the wrapper

(c) Bowie-Dick = air removal in pre-vacuum steam sterilizer; Helix/chemical = exposure parameters met; Biological (Geobacillus) = actual spore kill/gold-standard efficacy

(d) Biological indicators give instant results faster than chemical indicators

Answer: c

Solution: ✅ Mechanical/physical (gauges, printouts) + chemical (Bowie-Dick for air-steam mix, Class 5/6 integrators) + biological (Geobacillus stearothermophilus for steam) together validate a cycle.

🧠 Chemical shows exposure, not kill; only BI proves lethality — hence weekly + implant-load BI rule.

❌ Swapping steam/dry-heat organisms or expecting instant BI results confuses monitoring layers.

⚡ NORCET Fact: Steam BI — Geobacillus stearothermophilus; Dry heat/ETO BI — Bacillus atrophaeus; read BI after incubation (rapid readout ~1–3 hr).

Positive Marks: 1

Negative Marks: 0

Question: A steam load containing an orthopaedic implant shows a passed chemical integrator but the biological indicator later turns positive (growth). What is the correct action?

(a) Release the load since chemical passed

(b) Wipe implants with spirit and use anyway

(c) Ignore BI because chemical indicators are superior

(d) Recall and quarantine the load, do not use implants, investigate sterilizer, re-sterilize after corrective action with repeat BI negative before release

Answer: d

Solution: ✅ Positive BI = sterilization failure override — quarantine/recall, notify OT, service the sterilizer, reprocess and re-test until BI is negative.

🧠 Chemical pass only proves parameter exposure at the strip site; BI proves organism kill at the hardest-to-sterilize challenge.

❌ Releasing on chemical alone or spirit-wiping implants risks catastrophic prosthetic infection.

⚡ NORCET Fact: Hierarchy — BI result trumps chemical; every implant load needs BI; quarantine implant until BI result when feasible.

Positive Marks: 1

Negative Marks: 0

Question: Soiled instruments arrive in CSSD. Which unidirectional workflow prevents cross-contamination?

(a) Store sterile and dirty items together to save space

(b) Receiving/decontamination (dirty) → cleaning/washing → inspection/assembly/packaging (clean) → sterilization → sterile storage/distribution, with one-way dirty-to-clean traffic and separate zoning

(c) Clean packing area → wash/decontamination → sterile storage → wash again

(d) Sterilize first, then clean, then pack in the dirty zone

Answer: b

Solution: ✅ One-way dirty→clean flow with physical/temporal separation stops clean packs contacting aerosols, splashes and hands from the decontamination zone.

🧠 Progressive zoning (dirty, clean, sterile) plus hand-hygiene/PPE transitions enforces the gradient every shift.

❌ Reverse flow or mixed storage recontaminates sterile packs immediately.

⚡ NORCET Fact: CSSD mantra — Decontaminate → Clean → Inspect → Pack → Sterilize → Store → Issue; never cross back.

Positive Marks: 1

Negative Marks: 0

Question: During packaging for steam sterilization, which practice is correct?

(a) Use steam-permeable wrap/muslin/paper-pouch, place chemical indicator inside and outside, label with date/cycle/load, seal loosely enough for steam penetration and drying

(b) Seal in airtight plastic without sterilant-permeable wrap and secure with cello tape

(c) Overfill pouches and staple them shut for strength

(d) No labelling or indicator is needed for routine loads

Answer: a

Solution: ✅ Breathable packaging + internal + external indicators + intact seals + load traceability ensures penetration, exposure proof and recall ability.

🧠 Penetration needs permeable wrap and headspace; staples puncture barriers; indicators locate failures to specific packs.

❌ Cello tape melts and overfilled pouches block steam — both are classic packing failures.

⚡ NORCET Fact: Use indicator tape outside + integrator inside; record sterilizer number, cycle, date and pack contents for traceability.

Positive Marks: 1

Negative Marks: 0

Question: A nurse opens a sterile pack and notices the peel pouch has a 5 mm tear and the wrapper feels damp. What is the correct decision?

(a) Consider it unsterile — reject, reprocess/replace, and check storage/handling for causes of breach and wetting

(b) Use it if the expiry date is still valid

(c) Air-dry the pack and use it for a minor dressing

(d) Cover the tear with adhesive tape and use it

Answer: a

Solution: ✅ Any breach, dampness, soiling or crushed wrap breaks the sterile barrier — time-expiry alone never guarantees sterility.

🧠 Moisture wicks microbes through wrap (strike-through); tears create direct channels regardless of date.

❌ Taping or drying cannot restore validated sterility assurance.

⚡ NORCET Fact: Event-related sterility — sterile until an event (tear, wet, drop, dust, pest) compromises it, not merely until a date.

Positive Marks: 1

Negative Marks: 0

Question: Sterile instrument packs are stored in a closed cupboard 20 cm off the floor, 45 cm from the ceiling, away from sinks and direct sunlight at controlled temperature-humidity. What principle does this follow?

(a) Event-related sterility with contamination-risk minimisation through proper storage conditions and handling

(b) No storage rules apply to metal instruments

(c) Open-shelf storage on the floor is acceptable if packs are double-wrapped

(d) Time-related sterility only

Answer: a

Solution: ✅ Storage preserves sterility — closed cabinets, off-floor/away-from-ceiling clearances, low dust/humidity, minimal handling and FIFO rotation protect barrier integrity.

🧠 Floor/ceiling/sink zones concentrate dust, splashes and temperature swings — distance plus enclosure lowers event probability.

❌ Floor storage or ignoring environment invites wetting, pests and crush events even with double wrap.

⚡ NORCET Fact: Ideal store — ~20–24°C, 30–60% RH, 4–10 air exchanges, slotted/open shelving for airflow, handle with clean hands.

Positive Marks: 1

Negative Marks: 0

Question: A flexible endoscope manufacturer IFU specifies enzymatic cleaning, leak testing, brushing, specific HLD concentration-time and no ultrasonic use on a bonded joint. The technician skips brushing to save time. What is the correct judgment?

(a) Brushing is optional for all endoscopes

(b) Non-compliant — manufacturer IFU is mandatory; biofilm/bioburden left by skipping brushing blocks HLD contact and voids efficacy and warranty; repeat full IFU-compliant reprocessing

(c) Acceptable if HLD time is doubled without brushing

(d) Ultrasonic cleaning should be added despite IFU prohibition

Answer: b

Solution: ✅ IFU instructions are validated and legally binding — cleaning + brushing + leak test + compatible chemistry/time cannot be substituted or skipped.

🧠 Dried bioburden and biofilm physically shield microbes; unapproved ultrasonics can fracture bonded joints.

❌ Compensating with extra soak time without mechanical removal does not clear lumens.

⚡ NORCET Fact: Spaulding + IFU + guidelines rule — always follow the strictest of manufacturer instructions and institutional SOP; document traceability.

Positive Marks: 1

Negative Marks: 0